# M2M Wi-Fi Module

BT Provisioning Architecture Review

Release 0.5 (Draft — for discussion)

September 2026

# Document Revision History

| Version | Date | Changes |
| --- | --- | --- |
| 0.1 | 2026-09-26 | Initial draft — captures the BT/BLE provisioning architecture discussion: reusing Espressif's Unified Provisioning framework, the goal of also supporting third-party Wi-Fi chips (e.g. Realtek) with the same mobile app, and the recommended approach. |
| 0.2 | 2026-09-26 | Added §5.1: resolved the host-facing control-surface question (does the host MCU need visibility into BLE provisioning?) by adding AT*M2M*BT_PROV to the M2M-AT Command Set (Rev 1.5). |
| 0.3 | 2026-09-26 | Second review pass on §5.1's command: added a Query form, widened notifications to distinguish CRED_FAIL from a real timeout ERROR, decided auto-enabling WF_APMODE on success, made the window's non-persistence across reboot explicit, and renamed BT_PROV to BLE_PROV. Command Set bumped to Rev 1.5 accordingly (single bump covers both passes). |
| 0.4 | 2026-09-26 | Added §5.2: decided the BLE session only confirms the Wi-Fi join outcome; any downstream MQTT/AWS connection result goes through the existing host-facing AT*M2M*MQTT_CONN/AWS_CONN notifications instead of a new BLE-specific path. Command Set bumped to Rev 1.6 accordingly. Surfaced an open sub-question: whether every deployment actually has a host MCU listening. |
| 0.5 | 2026-09-26 | Added §5.3: documented a hardware-verified known limitation — only one AT*M2M*BLE_PROV session is supported per boot (AT*M2M*SYS_RST required before a further BLE_PROV=1 after a session has ended). Root cause: the ESP32-C3 BLE controller cannot be safely reinitialized once fully torn down within the same boot (verified crash inside Espressif's closed-source controller blob); mitigated by keeping the controller/NimBLE host alive across stop (wifi_prov_mgr_keep_ble_on()) so at least the crash is gone, though a second working session still isn't. Command Set bumped to Rev 1.7 accordingly. |

# 1. Background and Goals

This document is a working analysis, not a finalized spec — see the revision history. It covers the architecture question raised while planning BT/BLE-based provisioning for the M2M Wi-Fi module family (the same family M2M-AT Command Set.docx documents), and is meant to be read alongside that document rather than as a replacement for it.

Three requirements were stated together, and part of this document's job is to show where they pull in the same direction and where they don't:

- Add a BT/BLE path so an end-user's phone can deliver provisioning data (AP credentials and other setup information) to the module — this is a new, separate interface from the existing AT command set, not a replacement for it.

- Reuse Espressif's own Unified Provisioning framework as fully as possible for this.

- Keep the door open for third-party Wi-Fi chips (Realtek was named as an example) later — mirroring this project's own M2M-AT command set, which is defined once and re-implemented per chip's SDK.

- Reuse Espressif's own mobile app (not a custom-built app) for the phone side, as much as possible.

# 2. Espressif Unified Provisioning — What It Actually Is

"Unified Provisioning" is Espressif's umbrella term for a provisioning system built from two layers, both open-sourced as part of ESP-IDF. The distinction between these two layers is the key to the rest of this document:

| Layer | What it is | Portability |
| --- | --- | --- |
| protocomm | Session/transport layer: turns a raw BLE GATT link (or SoftAP HTTP, or a serial console) into a sequence of request/response "sessions", with an optional security handshake (Security0 = none, Security1 = X25519 key exchange + AES-CTR, Security2 = SRP6a PAKE + AES-GCM). | Wire protocol is chip-agnostic. The ESP-IDF C library implementing it is not. |
| wifi_provisioning | Defines the actual provisioning conversation as protobuf messages over protocomm "endpoints" (see the list below the table). | Wire protocol (the .proto schemas) is chip-agnostic. The ESP-IDF C library implementing it is not. |
| Mobile app | Espressif's official provisioning app (BLE and/or SoftAP transport) — a generic protocomm/protobuf client. It has no ESP32-specific logic; it only knows the wire protocol above. | Fully chip-agnostic by construction — this is the fact this whole document leans on. |

wifi_provisioning's three standard endpoints, for reference:

- prov-session — the protocomm security handshake itself

- prov-scan — trigger and read back a Wi-Fi scan

- prov-config — set STA credentials, read connection status

Custom app-specific data (e.g. this project's SYS_CONF-style fields, an AWS IoT endpoint, etc.) can ride the same session as an additional custom protocomm endpoint — Espressif's own examples (custom_config) show the pattern. That capability matters later (section 6).

# 3. Options Considered

## 3.1 Option A — Use Espressif Unified Provisioning unmodified, ESP32-only

Ship the stock ESP-IDF component and the stock app as-is. No wrapping, no abstraction.

- Pro: Least implementation effort; fully supported and security-reviewed by Espressif; the app works with zero custom work.

- Con: Doesn't address the stated multi-chip goal at all — there is no path to a Realtek version of this feature under this option.

## 3.2 Option B — Wrap BT provisioning in a custom command set, like AT commands

Define this project's own request/response protocol over BLE (an "AT-command-for-BT" analogue), the same way M2M-AT Command Set.docx defines one over UART/USB.

- Pro: Full ownership of the spec; genuinely chip-agnostic by design, same as the existing AT command set.

- Con: Breaks the "reuse Espressif's app" goal outright — that app is a fixed protocomm/protobuf client and cannot be reconfigured to speak a different wire protocol. Choosing this option means committing to building and maintaining a custom mobile app (iOS + Android, BLE stack, UX, app-store presence) — a materially larger, ongoing effort than the firmware side of this decision.

## 3.3 Option C (recommended) — Fix the wire protocol, re-implement it per chip

Treat Espressif's BLE GATT service/characteristic layout, the wifi_provisioning protobuf schemas, and the protocomm security handshake as a fixed specification — not "Espressif's library" but "the protocol Espressif's app speaks". ESP32-family builds satisfy that spec for free (the stock component already does). A future Realtek build satisfies the same spec by re-implementing it natively on Realtek's own BLE stack and Wi-Fi APIs — matching GATT UUIDs, protobuf messages, and the security handshake byte-for-byte.

This is structurally identical to how this project already treats M2M-AT: the command names/response formats are the portable spec; cmd_wifi.c (the ESP-IDF implementation) is one realization of it, and a Realtek port would be another realization of the same spec, not a new spec.

- Pro: Espressif's app keeps working, unmodified, against every chip that implements the spec — the actual goal stated at the start of this discussion.

- Pro: Consistent with this project's existing architecture philosophy (spec vs. implementation) — no new mental model to introduce.

- Con: The hard part of a Realtek port is re-implementing protocomm's security handshake (X25519 / SRP6a, AES-CTR / AES-GCM) protocol-compatibly — this is real cryptographic-protocol engineering, not a schema copy-paste. This is the actual cost driver of this option and should be scoped explicitly before committing to a Realtek timeline.

- Con: The spec is owned by Espressif, not this project — a future ESP-IDF release could change service UUIDs or protobuf message shapes. A specific ESP-IDF/component version needs to be pinned and tracked as "the spec version", the same way M2M-AT Command Set.docx pins a revision number.

## 3.4 Comparison Summary

|  | A: Stock, ESP32-only | B: Custom BT command set | C: Fixed spec, per-chip impl (recommended) |
| --- | --- | --- | --- |
| Espressif app reusable? | Yes | No — needs a custom app | Yes |
| Multi-vendor path? | No | Yes | Yes |
| New firmware effort per chip | None (ESP32 only) | Full custom protocol design once, then a per-chip implementation | Re-implement a fixed, already-defined spec per chip |
| New mobile-app effort | None | Full custom app (iOS + Android), ongoing | None |
| Hardest technical risk | None | Protocol design + app development capacity | protocomm security-handshake porting |

# 4. Recommendation

Adopt Option C. It is the only option that satisfies both stated hard requirements at once (reuse Espressif's app; support additional Wi-Fi chip vendors later) — Option A drops the multi-vendor goal, Option B drops the app-reuse goal. The cost is concentrated in one well-defined, scopeable piece of engineering (the protocomm security-handshake port) rather than spread across an entire custom protocol *and* a custom app, which is why it is also the lower-risk option of the two that actually meet both goals.

# 5. Relationship to the Existing AT Command Set

BT/BLE provisioning and the AT command set are separate interfaces serving different actors, and should stay that way rather than merging:

|  | AT Command Set | BT/BLE Provisioning |
| --- | --- | --- |
| Other party | The host MCU / integrator's application | The end user's phone, during initial setup |
| Transport | UART / native USB Serial-JTAG | BLE (this document); SoftAP/HTTP also possible under the same framework |
| Protocol ownership | This project (M2M-AT Command Set.docx) | Espressif's spec (Option C, section 3.3) |
| Lifecycle | Present for the device's whole operating life | Primarily a one-time (or occasional re-) setup flow |

The two should still meet at one point internally: whatever BLE provisioning receives (SSID/password, and any custom fields added per section 2's last paragraph) should be applied through the same internal functions the AT command set's own handlers already call (e.g. cmd_wf_conn()'s underlying Wi-Fi configuration logic in cmd_wifi.c), not a second, parallel Wi-Fi-configuration code path. This keeps there being exactly one place that actually knows how to join a network, regardless of which interface asked for it.

## 5.1 Host-Facing Control Surface — Resolved

A follow-up question during this review: does the host MCU need to know about any of Espressif's BLE/provisioning commands? The answer split in two:

- No — everything internal to the BLE leg (GATT services/characteristics, protocomm sessions, the wifi_provisioning protobuf messages themselves) stays fully hidden from the host, exactly like today's Wi-Fi driver internals are hidden behind cmd_wifi.c.

- Yes — two product-level touchpoints are still needed at the host-facing AT layer: (1) the host plausibly needs to control when the BLE provisioning window is open (it shouldn't just be on forever — battery and pairing-window security both argue for the host driving this, e.g. off a physical button), and (2) the host needs to know when a phone has provisioned the device over BLE, the same way it already learns about a Wi-Fi join it didn't itself request.

Resolved by adding AT*M2M*BLE_PROV to the M2M-AT Command Set (§3.5, Rev 1.5). A second review pass caught five further gaps in the first cut of this command, all now folded into the same spec:

- Query form added — AT*M2M*BLE_PROV=? reports whether the window is open and, if so, seconds remaining. The window is a persistent piece of state (like WF_MODE), not a one-shot action like WF_WPS, so it needed one even though WF_WPS itself doesn't have one.

- Notification set widened from a single generic error into three distinct outcomes the host can actually act on differently: IND CONNECTED (a phone attached over BLE), IND CRED_RECEIVED (it sent Wi-Fi credentials), IND CRED_FAIL (those credentials failed to join — window stays open for the phone to retry, mirroring Espressif's own wifi_prov_mgr behavior), DONE (joined successfully), and ERROR 1 only for the window timing out with nobody completing setup — a host-issued close (BLE_PROV=0) is not an error, it's just acknowledged by the command's own OK and nothing else.

- Persistence decided explicitly: on DONE, the module also enables AT*M2M*WF_APMODE=1 itself, so the phone-provisioned connection survives a reboot without the host having to do anything — appropriate because BLE provisioning is normally a one-time end-user step, unlike a host-driven WF_CONN where the host is expected to manage WF_APMODE itself.

- Reboot behavior for the window itself made explicit: it never persists — always starts closed (state=0) after a reset, regardless of what it was set to before.

- Renamed BT_PROV → BLE_PROV — the whole discussion (and the underlying Espressif framework) is BLE-specific; the old name risked reading as classic Bluetooth (BR/EDR), which this has nothing to do with.

Deliberately not resolved this pass — carried forward to §6: whether non-Wi-Fi custom data (an AWS endpoint, a customer code, etc.) should ride the same BLE session. AT*M2M*BLE_PROV as specified only covers Wi-Fi credentials; extending it is a separate follow-up once the custom-protocomm-endpoint question below is actually decided.

## 5.2 BLE Session Scope — Wi-Fi Confirmation Only

A related question: once a phone has handed over Wi-Fi credentials (and, later, possibly MQTT/AWS config over a custom endpoint per §6), should the module also connect to those downstream servers while the phone is still there, and report that result back over BLE too — or is confirming the Wi-Fi join enough?

Decided: Wi-Fi join confirmation only. The BLE session's job ends at *M2M*BLE_PROV:DONE (or ERROR). Whatever MQTT/AWS/other server connection the delivered config later enables happens afterward, over the now-established Wi-Fi link, and is reported through the existing host-facing notification path (AT*M2M*MQTT_CONN/AT*M2M*AWS_CONN's own :DONE/:ERROR) — the same path a host-initiated connection already uses, not a new BLE-specific one.

Reasoning:

- Latency and session fragility: MQTT/AWS setup (TLS handshakes, and for AWS specifically this project's own Fleet Provisioning by Claim round trip) can take far longer than a phone user will wait with the BLE screen open, and the BLE link itself is more likely to drop the longer it stays connected.

- Failure-domain separation: "the Wi-Fi password was wrong" and "the MQTT certificate was wrong" are different problems a user or integrator troubleshoots differently — collapsing both into one BLE-session outcome makes both harder to diagnose.

- No new mechanism needed: this project already has a working notification path for exactly this (Chapters 6.4/6.5's :DONE/:ERROR messages) — reusing it instead of inventing a BLE-specific equivalent is both less code and one fewer thing to keep in sync with the Command Set spec.

Open sub-question this decision surfaces (tracked in §6): does every deployment of this device have a host MCU listening on the AT interface? If a phone-only deployment (no host) is ever planned, nothing currently reports MQTT/AWS outcomes anywhere reachable by the end user once BLE has disconnected — that gap would need a cloud-side answer (e.g. a device shadow the app polls), not a BLE-session one.

## 5.3 Known Limitation: Single BLE Session Per Boot

Hardware-verified (2026-09-26, real ESP32-C3 device on this project's own build): once an AT*M2M*BLE_PROV session has ended — BLE_PROV=0, a timeout, or a start failure — a further BLE_PROV=1 in the same power cycle does not reliably open a new session. AT*M2M*SYS_RST is required first. This is now documented as a known limitation in the M2M-AT Command Set (Rev 1.7, section 3.5).

Root cause: wifi_provisioning's BLE scheme owns the ESP32-C3 BLE controller and NimBLE host for the duration of a session. Its NimBLE start routine (protocomm_nimble.c's simple_ble_start()) always calls nimble_port_init() unconditionally — it has no path to resume an already-running controller/host for a fresh session. That leaves two ways to end a session, and both were tried:

• Fully deinit the controller on stop (this project's original design). The next wifi_prov_mgr_init()+start_provisioning() call then crashes: a NULL function-pointer call inside Espressif's closed-source BLE controller blob (r_lld_env_init, resolved via addr2line against the build's own ELF), because the ESP32-C3 BLE controller cannot be safely reinitialized once fully torn down within the same boot.

• Keep the controller/NimBLE host alive across stop instead (wifi_prov_mgr_keep_ble_on(1), plus the CONFIG_WIFI_PROV_KEEP_BLE_ON_AFTER_PROV/CONFIG_WIFI_PROV_DISCONNECT_AFTER_PROV Kconfig options) — what the firmware actually does now. BLE_PROV=0 still stops advertising and disconnects any connected peer, so the doc's “provisioning window closed” contract holds and the device is not left discoverable/connectable. But the next simple_ble_start() then fails cleanly at esp_bt_controller_init() (ESP_ERR_INVALID_STATE, since the controller is already up) instead of crashing — a real improvement (no crash, ever), but still not a working second session.

A working second in-boot session would mean bypassing wifi_prov_mgr's BLE scheme entirely and driving protocomm/NimBLE directly — registering and unregistering just the GATT service on the already-running host, without ever touching the controller after its first init. That is a materially larger change than this command's original scope and is not planned; a device reset between BLE provisioning attempts is an acceptable cost given how infrequently this path is used in practice (initial commissioning, or a deliberate re-provision).

# 6. Open Items for Follow-Up

- Pin the exact ESP-IDF / wifi_provisioning / protocomm component versions this project targets as "the spec", the way M2M-AT Command Set.docx pins its own revision.

- Decide whether any non-Wi-Fi data needs to ride the same provisioning session (e.g. an AWS IoT endpoint, a customer code, or other SYS_CONF-style fields) — if so, design the custom protocomm endpoint for it now, rather than after a Realtek port is underway. Partially resolved (§5.2): whatever this decision lands on, the BLE session itself will only ever report the Wi-Fi join outcome, not any downstream MQTT/AWS result — that still goes through the existing host-facing AT notifications.

- New from §5.2: confirm whether every real deployment of this device has a host MCU on the AT interface. If a phone-only (no host) deployment is ever planned, add a cloud-side status mechanism (e.g. a device shadow) for MQTT/AWS outcomes — the BLE session deliberately doesn't cover this.

- Scope the Realtek port specifically: confirm Realtek's SDK BLE GATT server API surface, and get an early read on how hard a protocol-compatible protocomm security-handshake port actually is before committing to a timeline.

- Confirm whether the mobile app (or the user) needs any way to tell which chip vendor a given device uses — if the wire protocol is truly identical, this may not matter at all, but it's worth confirming rather than assuming.

- Decide how a provisioned device's BLE service is discovered/scoped for security (device-name pattern, proof-of-possession, QR code — Espressif's app supports several) — this is independent of the chip-portability question but needs an answer before any real deployment.

- Resolved this revision (see §5.1): the host-facing control surface question — AT*M2M*BLE_PROV (Query+Set, full notification set, persistence and reboot behavior all specified) added to the Command Set.

# Appendix A. References

- ESP-IDF Wi-Fi Provisioning: docs.espressif.com/projects/esp-idf/en/stable/esp32c3/api-reference/provisioning/wifi_provisioning.html

- ESP-IDF protocomm: docs.espressif.com/projects/esp-idf/en/stable/esp32c3/api-reference/provisioning/protocomm.html

- Related project document: M2M-AT Command Set.docx (this project's own host-facing command protocol, Rev 1.4)