M2M Wi-Fi Module

M2M-AT Command Set

User Guide

Release 1.0

July 2026

Document Revision History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | 2026-08 | Initial release |
| 1.1 | 2026-09-14 | Added AT*M2M*WF_IPSTATUS Set Command (static station IP), AT*M2M*NET_FILELIST, AT*M2M*NET_FILEINFO, and AT*M2M*MQTT_STATUS -- gaps found while building the configuration web server's IP Settings, TLS Certificates, and MQTT Broker screens. |
| 1.2 | 2026-09-14 | AT*M2M*NET_CONN's <remote_ip> renamed <remote_host> and documented to accept a hostname (resolved via DNS) in addition to a literal IPv4 address, for TCP/UDP/SSL alike -- previously TCP/UDP silently rejected anything but a literal IP. |
| 1.3 | 2026-09-14 | AT*M2M*AWS_PAIR (and AT*M2M*AWS_CERT's op=FETCH, which depends on it) marked deprecated in favor of AWS IoT's standard Fleet Provisioning by Claim (AT*M2M*AWS_CLAIMCERT + AT*M2M*AWS_PROVISION) -- the custom pairing/certificate-server protocol AWS_PAIR depends on was never specified anywhere in this document and isn't implementable against a real service. Also fixed *M2M*NET_IP's Message Command/Example reading "*M2M*IP:IND" instead of "*M2M*NET_IP:IND" (Ch.7.2), inconsistent with its own heading, the Ch.7.1 summary table, and the implementation. |
| 1.4 | 2026-09-26 | Version strings aligned with M2M S/W Release Naming Guide 2.0: AT*M2M*SYS_VER and AT*M2M*OTA_CHECK report the software version in the fixed <MM>.<NN> format (e.g. 01.00), separate from the release file name; AT*M2M*SYS_VER appends a <customer_code> field (e.g. AC0); OTA_CHECK documents the comparison rule; AT*M2M*OTA_UPDATE adds reason 5 (downloaded image ID does not match the installed image). Also fixed three internal inconsistencies found while implementing this revision: SYS_LSLEEP's Query response didn't echo <mode> (every sibling Query/Set command does); SYS_LSLEEP's mode 3 used a colon instead of a hyphen; SYS_ANTENNA's Set example didn't echo <type>, inconsistent with its own documented Response format. |
| 1.5 | 2026-09-26 | Added AT*M2M*BLE_PROV (§3.5): a Query+Set command to check/start/stop a BLE provisioning window (Espressif Unified/BLE Provisioning-compatible) so an end-user's phone app can deliver Wi-Fi credentials directly to the module without going through the host. On success, the module also enables AT*M2M*WF_APMODE=1 so the connection survives a reboot; the provisioning window itself never persists across a reset. Added the corresponding *M2M*BLE_PROV:IND(CONNECTED/CRED_RECEIVED/CRED_FAIL)/DONE/ERROR notifications to Ch.7.1. See the companion M2M BT Provisioning Architecture Review document for the design rationale (multi-vendor Wi-Fi chip support while keeping Espressif's own mobile app usable). |
| 1.6 | 2026-09-26 | Scoped AT*M2M*BLE_PROV to Wi-Fi credentials only: the BLE session confirms Wi-Fi join success/failure and nothing else. Any MQTT/AWS/other server connection the delivered credentials enable is established afterward over the now-up Wi-Fi link and reported the same way a host-initiated connection already is (AT*M2M*MQTT_CONN/AWS_CONN notifications) -- not part of the BLE session. See the companion Architecture Review, section 5.1, for the reasoning (latency, mixing unrelated failure domains, and reusing the existing host-facing notification path instead of inventing a second one). |
| 1.7 | 2026-09-26 | Documented a hardware-verified known limitation of AT*M2M*BLE_PROV (section 3.5): only one provisioning session is supported per boot -- AT*M2M*SYS_RST is required before a further BLE_PROV=1 after a session has ended (BLE_PROV=0, a timeout, or a start failure), or it fails with ERROR 0. See the companion Architecture Review, section 5.3, for the root cause (the ESP32-C3 BLE controller cannot be safely reinitialized within the same boot once fully torn down). |
| 1.8 | 2026-09-27 | EN 18031-1 (RED cybersecurity) hardening: AT*M2M*NET_HTTPDCONF now stores only a salted password hash, reports ******** in place of the password, enforces a password policy (new reason 1), and the factory default admin/admin must be changed at the first web login; web login has brute-force lockout. AT*M2M*NET_HTTPDSTART adds reason 4 (transport disabled in this firmware build). AT*M2M*WF_APMODE and AT*M2M*WF_EAPCONF Query responses mask stored passwords. AT*M2M*WF_APSTART may refuse auth=0 (open SoftAP) in builds without open-SoftAP support. New AT*M2M*BLE_POP (§3.5): per-module BLE provisioning proof-of-possession, replacing the fixed value every module shared. |
| 1.9 | 2026-10-04 | AT*M2M*NET_HTTPDOWNLOAD: stated the module file system capacity (about 228 KB in total), which was reduced to make room for firmware growth. AT*M2M*WF_CONN / WF_APMODE: an authentication failure is now retried up to 3 times (3 s apart) before *M2M*WF_DISCONN:DONE is reported. |
| 1.10 | 2026-10-04 | EN 18031-1 Phase C. TLS: TLS 1.3 added; key exchanges without forward secrecy (RSA, static ECDH) and renegotiation removed, for every TLS client and server of the module. AT*M2M*NET_HTTPDOWNLOAD: certificates/keys weaker than RSA-2048 / EC P-256 are refused (reason 5); file names limited to 1-30 characters of letters, digits, '.', '_', '-' (reason 2). AT*M2M*WF_CONN / WF_APSTART: an SSID over 32 bytes or a passphrase outside 8-64 characters is refused (reason 2) instead of being truncated; AT*M2M*MQTT_CONF likewise for over-long fields, a port outside 1-65535 or an invalid cert_name; AT*M2M*WF_IPSTATUS refuses a malformed address. WPA2/WPA3-Enterprise: with no CA certificate staged, the RADIUS server certificate is now verified against the built-in public CA bundle (a server signed by a private CA needs its CA staged with AT*M2M*WF_EAPCERT type 2). AT*M2M*NET_SERVER (ssl): a client that does not finish the TLS handshake within 10 s is closed (*M2M*NET_CLI_CLOSED). |
| 1.11 | 2026-10-04 | Flash-stored parameters (Wi-Fi credentials, login data, BLE PoP, broker settings) are now encrypted with a per-device key held in the chip. Private-key files are kept in that encrypted storage rather than the plain file system: AT*M2M*NET_HTTPDOWNLOAD and the web upload refuse a private-key file larger than 8 KB (reason 5), and AT*M2M*SYS_FACTORY now also deletes stored private keys. Updating a module from an earlier firmware clears the stored parameters once. |

Table of Contents

Document Revision History	2

Table of Contents	3

1. Introduction	4

1.1 Command Conventions	4

1.2 Command Types	4

1.3 Synchronization and Notification Messages	4

1.4 Byte Stuffing for Binary Payloads	5

2. Basic AT Commands	6

3. Wi-Fi AT Commands	11

3.1 Common Commands	11

3.2 Station Commands	12

3.3 SoftAP Commands	14

3.4 WPS	15

3.5 BLE Provisioning	15

4. TCP/IP AT Commands	17

4.1 Command Mode	17

4.2 Socket and Address Commands	17

6. Network Service AT Commands	22

6.1 DHCP Server / Client	22

6.2 HTTP Client	23

6.3 HTTP Server	26

6.4 MQTT Client	27

6.5 AWS Client	29

6.6 Firmware OTA	33

7. Notification Messages	35

7.1 Message Summary	35

7.2 Message Details	36

8. Usage Examples	38

8.1 Joining an Access Point	38

8.2 Running a SoftAP	38

8.3 TCP and UDP Sockets	39

8.4 TLS Connections	39

8.5 Entering and Leaving Data Mode	40

8.6 MQTT Broker and AWS IoT	40

Appendix A. CONF Reference	42

A.1 Readable Attributes (AT*M2M*SYS_CONF)	42

A.2 Writable Attributes (AT*M2M*SYS_CONF)	42

Appendix B. Country Codes and Channel Plans	43

Appendix C. Error Codes	44

1. Introduction

The M2M Wi-Fi module is a serial-to-Wi-Fi bridge. A host MCU attached over UART drives the module with a plain-text AT command set and, through it, gains IEEE 802.11 connectivity together with a complete TCP/IP stack — sockets, DHCP, DNS, SNTP, HTTP, SSL/TLS and related services — without running any wireless or network software of its own. The RTOS, Wi-Fi protocol stack, supplicant and TCP/IP stack are all embedded in the module firmware.

Because the host only needs a UART and a small command parser, even 8-bit microcontrollers can add wireless networking with minimal integration effort. Typical applications include building and factory automation, energy management, industrial sensor networks, medical devices, remote monitoring and control, security systems, and transportation.

This AT command set was independently developed for the M2M Wi-Fi module. Its command structure follows conventions common to serial-to-Wi-Fi AT command interfaces, so that host MCU software already built around a similar command syntax can be ported to this module with minimal changes.

1.1 Command Conventions

Throughout this guide the following notation is used:

- < > encloses a mandatory parameter.

- [ ] encloses an optional parameter.

- Parameters are separated by a single space character.

- Every command sent by the host starts with "AT" and is terminated with a carriage return, <CR> (0x0D).

- Every response and every notification message sent by the module is terminated with <CR><LF> (0x0D 0x0A).

- Commands and responses carry the vendor tag *M2M* after the leading "AT" — a command line reads AT*M2M*<NAME>, the matching response reads *M2M*<NAME>:OK or *M2M*<NAME>:ERROR [code].

Command echo is disabled by default; use ATE1 to see typed characters on a terminal and ATE0 to turn the echo off again.

1.2 Command Types

| Type | Format | Function |
| --- | --- | --- |
| Test | AT*M2M*<NAME>? | List the parameter format and range accepted by the Set form. |
| Query | AT*M2M*<NAME>=? | Read the current value or state. |
| Set | AT*M2M*<NAME>=<p1> [p2] ... | Write configuration or trigger an action with parameters. |
| Execute | AT*M2M*<NAME> | Run an action that takes no parameters. |

Not every command implements all three types; each reference entry lists the forms it supports.

1.3 Synchronization and Notification Messages

The interface is half-duplex at the protocol level: after sending a command, the host must wait for the matching response before issuing the next command. Independently of this request/response flow, the module raises unsolicited notification messages (for example *M2M*CONNECTED or *M2M*RECV) whenever an asynchronous event occurs. Chapter 7 lists all messages.

1.4 Byte Stuffing for Binary Payloads

Payload bytes passed inside AT*M2M*SEND and AT*M2M*SENDTO may collide with characters the command parser treats specially. The host must therefore escape the three bytes below before placing them in a payload field; the module transparently restores the original bytes before transmission, and never applies stuffing in the module-to-host direction. The host can rely on the size field of a message to tell payload bytes apart from control characters.

| Byte | Meaning | Escaped sequence (0x1B, byte XOR 0x20) |
| --- | --- | --- |
| 0x0D | Carriage return | 0x1B 0x2D |
| 0x08 | Backspace | 0x1B 0x28 |
| 0x1B | Escape | 0x1B 0x3B |

2. Basic AT Commands

AT

Tests whether the host and the module can talk to each other. Use this as the very first command after power-up or a baud-rate change.

Execute Command:

AT

Response:

OK

Example:

AT

<- OK

ATE

Turns command echo on or off. With echo enabled, every character the host sends is mirrored back, which is convenient when typing on a terminal.

Set Command:

ATE<0|1>

Response:

OK

// or

ERROR

Parameters:

0 : echo off (default)

1 : echo on

Example:

ATE1

<- OK

AT*M2M*SYS_VER

Reads the firmware version of the module.

Query Command:

AT*M2M*SYS_VER=?

Response:

*M2M*SYS_VER:OK <version> <compile time> <customer_code>

Parameters:

version: software version <MM>.<NN>, fixed five characters (MM 00-99, NN 00-99), e.g. 01.00

compile time: compile time

customer_code: three-character customer code the image was built for (two-letter customer code + release type), e.g. AC0 = standard ESP32-C3 board default. Not part of <version> and not used for version comparison

Example:

AT*M2M*SYS_VER=?

<- *M2M*SYS_VER:OK 01.00 2026-09-26-14:02:11 AC0

AT*M2M*SYS_MAC

Reads the MAC address of the module.

Query Command:

AT*M2M*SYS_MAC=?

Response:

*M2M*SYS_MAC:OK <mac>

Parameters:

mac : MAC address, formatted aa:bb:cc:dd:ee:ff

Example:

AT*M2M*SYS_MAC=?

<- *M2M*SYS_MAC:OK 84:72:07:01:02:03

AT*M2M*SYS_RST

Restarts the module.

Set Command:

AT*M2M*SYS_RST=<cmd>

Response:

*M2M*SYS_RST:OK

Parameters:

cmd: 0-restart the module

Example:

AT*M2M*SYS_RST=0

<- *M2M*SYS_RST:OK

AT*M2M*SYS_FACTORY

Restores all flash-stored parameters to factory defaults and restarts the module. Stored private-key files (see AT*M2M*NET_HTTPDOWNLOAD) are deleted as well; certificates and other files are kept.

Set Command:

AT*M2M*SYS_FACTORY=<cmd>

Response:

*M2M*SYS_FACTORY:OK

Parameters:

cmd: 0-initialize NV items; 1-initialize NV items and restart

Example:

AT*M2M*SYS_FACTORY=0

<- *M2M*SYS_FACTORY:OK

AT*M2M*SYS_LSLEEP

Reads or sets the power-save policy.

Query Command:

AT*M2M*SYS_LSLEEP=?

Response:

*M2M*SYS_LSLEEP:OK <mode>

Set Command:

AT*M2M*SYS_LSLEEP=<mode>

Response:

*M2M*SYS_LSLEEP:OK

// or

*M2M*SYS_LSLEEP:ERROR [error_code]

Parameters:

mode: 0-disabled; 1-modem sleep (DTIM-based); 2-light sleep; 3-modem sleep (listen-interval)

reason: 0-not supported

Example:

AT*M2M*SYS_LSLEEP=1

<- *M2M*SYS_LSLEEP:OK

AT*M2M*SYS_DSLEEP

AT*M2M*SYS_ANTENNA

Reads & Set which antenna the radio is currently configured to use.

Query Command:

AT*M2M*SYS_ANTENNA=?

Response:

*M2M*SYS_ANTENNA:OK <type>

// or

*M2M*SYS_ANTENNA:ERROR [error_code]

Set Command:

AT*M2M*SYS_ANTENNA=<type>

Response:

*M2M*SYS_ANTENNA:OK <type>

// or

*M2M*SYS_ANTENNA:ERROR [error_code]

Parameters:

type: 0-on-board chip antenna; 1-external antenna

reason: 0-not supported

Example:

AT*M2M*SYS_ANTENNA=1

<- *M2M*SYS_ANTENNA:OK 1

AT*M2M*SYS_UART

Reads or sets the UART configuration saved to flash and used after every reset.

Query Command:

AT*M2M*SYS_UART=?

Response:

*M2M*SYS_UART:OK <baud_rate>

// or

*M2M*SYS_UART:ERROR [error_code]

Set Command:

AT*M2M*SYS_UART=<baud_rate>

Response:

*M2M*SYS_UART:OK <baud_rate>

// or

*M2M*SYS_UART:ERROR [error_code]

Parameters:

baud_rate: Specifies the UART baud rate in bits per second (bps) (default 115200)

reason: 0-not supported

Example:

AT*M2M*SYS_UART=115200

<- *M2M*SYS_UART:OK 115200

AT*M2M*SYS_CONF

Reads & set one attribute of modem – Appendix A.1 lists every readable index.

Query Command:

AT*M2M*SYS_CONF=<cmd> <index>

Response:

*M2M*SYS_CONF:OK <value>

// or

*M2M*SYS_CONF:ERROR [error_code]

Set Command:

AT*M2M*SYS_CONF=<cmd> <index> <value>

Response:

*M2M*SYS_CONF:OK

// or

*M2M*SYS_CONF:ERROR [error_code]

Parameters:

cmd: 0-get; 1-set

index: See Appendix A.1

value: See Appendix A.1

reason: 0-not supported

Example:

AT*M2M*SYS_CONF=0 1

<- *M2M*SYS_CONF:OK 01:02:03:01:02:03

AT*M2M*SYS_CONF=1 35 wifimodule

<- *M2M*SYS_CONF:OK

// or (Use "" for special characters.)

AT*M2M*SYS_CONF=1 35 “wifi_module \””

<- *M2M*SYS_CONF:OK

AT*M2M*SYS_COUNTRY

Reads & set one attribute of modem – Appendix B.1 lists every readable index.

Query Command:

AT*M2M*SYS_COUNTRY=?

Response:

*M2M*SYS_COUNTRY:OK <country_code>

// or

*M2M*SYS_COUNTRY:ERROR [error_code]

Set Command:

AT*M2M*SYS_COUNTRY=[country_code]

Response:

*M2M*SYS_COUNTRY:OK

// or

*M2M*SYS_COUNTRY:ERROR [error_code]

Parameters:

country_code: See Appendix B.1

reason: 0-not supported

Example:

AT*M2M*SYS_COUNTRY=?

<- *M2M*SYS_COUNTRY:OK KR

AT*M2M*SYS_COUNTRY=US

<- *M2M*SYS_COUNTRY:OK

3. Wi-Fi AT Commands

The commands in this chapter configure the radio and manage station, SoftAP, WPS operation.

3.1 Common Commands

AT*M2M*WF_MODE

Reads or sets the Wi-Fi mode.

Query Command:

AT*M2M*WF_MODE=?

Response:

*M2M*WF_MODE:OK <mode>

Set Command:

AT*M2M*WF_MODE=<mode> [auto_connect]

Response:

*M2M*WF_MODE:OK

// or

*M2M*WF_MODE:ERROR [error_code]

Parameters:

mode: 0-init; 1:station; 2-SoftAP 3-station+SoftAP

auto_connect: 0-disable; 1:enable

reason: 0-not supported

Example:

AT*M2M*WF_MODE=2

<- *M2M*WF_MODE:OK

AT*M2M*WF_MODE=2 1

<- *M2M*WF_MODE:OK

AT*M2M*WF_APMODE

Controls automatic reconnection. When enabled, the module stores its working connection profile in NV memory and rejoins the network by itself after a reboot or a link loss, according to the selected policy.

Activates predefined station profile.(soft AP or station)

Query Command:

AT*M2M*WF_APMODE=?

Response:

*M2M*WF_APMODE:OK [type] [ssid] [channel] {password}

Set Command:

AT*M2M*WF_APMODE= [type] [ssid] [channel] {password}

Response:

*M2M*WF_APMODE:OK

Parameters:

type: 0-init; 1- start SoftAP with the predefined SSID, channel and security; 2- start SoftAP with the SSID given in ssid, channel and password

reason: 0-not supported

password: in the Query response, ******** when a password is stored (the stored password itself is never returned)

- Example:

- AT*M2M*WF_APMODE=1

- <- *M2M*WF_APMODE:OK

- AT*M2M*WF_APMODE=2 “m2m_ap” 11

- <- *M2M*WF_APMODE:OK

3.2 Station Commands

AT*M2M*WF_SCAN

Scans all permitted channels for nearby networks.

Execute Command:

AT*M2M*WF_SCAN

Response:

*M2M*WF_SCAN:OK

*M2M*WF_SCAN:IND <ssid> <rssi> <bssid> <channel> <auth> // one line per network

*M2M*WF_SCAN:DONE

// or

*M2M*WF_SCAN:ERROR [error_code]

Parameters:

auth: 0-open; 1: WEP; 2: WPA-PSK; 3: WPA2-PSK; 4: WPA/WPA2-PSK, 5: WPA/WPA3-PSK-Mixed, 6: WPA3

reason: 0-not supported

AT*M2M*WF_CONN

Joins an access point in station mode

Query Command:

AT*M2M*WF_CONN=?

Response:

*M2M*WF_CONN:OK <ssid> <bssid> <channel> <rssi>

// or

*M2M*WF_CONN:ERROR [error_code]

Set Command:

AT*M2M*WF_CONN=<ssid> [password] [bssid] [wep_key_index]

Response:

*M2M*WF_CONN:OK

// or

*M2M*WF_CONN:ERROR [error_code]

Parameters:

ssid: up to 32 characters. Wrap in double quotes (ASCII 0x22) if it contains spaces; escape an embedded double quote with a slash "/" (ASCII 0x2F

password: WEP: 10/26 hex digits or 5/10 ASCII characters. WPA/WPA2/WPA3: 8–64 ASCII characters. The same quoting and escaping rules as for ssid apply.

bssid: MAC address of a specific AP to join

wep_key_index: 1–4; only meaningful for WEP, default 1

reason: 0-not supported

Notes:

The result also arrives asynchronously as *M2M*WF_CONN:DONE / *M2M*WF_CONN:IND

If the AP rejects the authentication (for example because it still holds the association of this module from before a power loss), the module retries the join up to 3 times, 3 seconds apart, without reporting the intermediate failures; *M2M*WF_DISCONN:DONE is sent only if the last retry also fails. A wrong password or an AP that is not found is reported immediately. AT*M2M*WF_DISCONN cancels a pending retry. The same applies to the automatic reconnection of AT*M2M*WF_APMODE.

Example:

// open network

AT*M2M*WF_CONN=m2m_ap

<- *M2M*WF_CONN:OK

// then, asynchronously

<- *M2M*WF_CONN:DONE

// WPA2, passphrase with a space

AT*M2M*WF_CONN=m2m_ap "1234 5678"

<- *M2M*WF_CONN:OK

// then, asynchronously

<- *M2M*WF_CONN:DONE

// pin to a specific BSSID

AT*M2M*WF_CONN=m2m_ap 12345678 00:01:02:03:04:05

// then, asynchronously

<- *M2M*WF_CONN:DONE

// space SSID

AT*M2M*WF_CONN=”m2m_ap @/@” “12345678 \## 00” 00:01:02:03:04:05

<- *M2M*WF_CONN:OK

// then, asynchronously

<- *M2M*WF_CONN:DONE

AT*M2M*WF_DISCONN

Disconnects from the current access point.

Execute Command:

AT*M2M*WF_DISCONN

Response:

*M2M*WF_DISCONN:OK

// or

*M2M*WF_DISCONN:ERROR [error_code]

Parameters:

reason: 0-not supported

Example:

AT*M2M*WF_DISCONN

<- *M2M*WF_DISCONN:OK

AT*M2M*WF_EAPCONF

Configures the credentials for WPA/WPA2/WPA3-Enterprise (802.1X/EAP) authentication.

Query Command:

AT*M2M*WF_EAPCONF=<cmd> <method>

Response:

*M2M*WF_EAPCONF:OK <method> <id> <password>

// or

*M2M*WF_EAPCONF:ERROR [error_code]

Set Command:

AT*M2M*WF_EAPCONF=<cmd> <method> <id> <password>

Response:

*M2M*WF_EAPCONF:OK

// or

*M2M*WF_EAPCONF:ERROR [error_code]

Parameters:

cmd: 0-get; 1-set

method: tls, ttls, peap, leap or fast

id : user identity (max. 64 ASCII characters), as provisioned on the RADIUS server / anonymous identity (max. 64 characters), used by TTLS and FAST

password: private-key password for the TLS client certificate (max. 128 characters) / EAP password for TTLS/PEAP/LEAP/FAST (max. 128 characters). The get (cmd=0) response reports ******** instead of the stored password.

reason: 0-not supported

Example:

// PEAP with username and password

AT*M2M*WF_EAPCONF=0 peap "user" "password"

<- *M2M*WF_EAPCONF:OK

AT*M2M*WF_EAPCERT

Configures the certificate files and inner method for WPA/WPA2/WPA3-Enterprise authentication. The referenced files must already be stored on the module

Set Command:

AT*M2M*WF_EAPCERT=<type> <value> {..<type> <value>}

Response:

*M2M*WF_EAPCERT:OK

// or

*M2M*WF_EAPCERT:ERROR [error_code]

Parameters:

type: 0- phase-2 method, MSCHAPv2; 1- PEAP phase-1 label; 2- CA (root) certificate file name (max. 16 characters), 3- TLS client certificate file name (max. 16 characters), 4- TLS private-key file name (max. 16 characters); 5- FAST PAC file name (max. 16 characters)

reason: 0-not supported

Example:

// TLS with full certificate set

AT*M2M*WF_CERT=2 ca.pem 3 tls.pem 4 key.pem

<- *M2M*WF_CERT:OK

AT*M2M*WF_IPSTATUS

Reads or sets the station / SoftAP IP configuration.

Query Command:

AT*M2M*WF_IPSTATUS=?

Response:

*M2M*WF_IPSTATUS:OK <bss_type> <dhcp> <ip> <subnet> <gateway>

// or

*M2M*WF_IPSTATUS:ERROR [error_code]

Parameters:

bss_type: 1: station; 2: softAP; 3-adhoc; 4-mesh

dhcp: 0-static; 1: DHCP

reason: 0-not supported

Example:

AT*M2M*WF_IPSTATUS=?

<- *M2M*WF_IPSTATUS:OK 1 1 192.168.0.68 255.255.255.0 192.168.0.1

Set Command:

AT*M2M*WF_IPSTATUS=<dhcp> [ip] [subnet] [gateway]

Response:

*M2M*WF_IPSTATUS:OK

// or

*M2M*WF_IPSTATUS:ERROR [error_code]

Parameters:

dhcp: 0-static (ip/subnet/gateway required); 1-DHCP (ip/subnet/gateway ignored)

Applies to the station interface only; SoftAP addressing is configured via AT*M2M*WF_APSTART / AT*M2M*NET_DHCPS.

reason: 0-not supported, 2-invalid IP arguments

Example:

AT*M2M*WF_IPSTATUS=0 192.168.1.100 255.255.255.0 192.168.1.1

<- *M2M*WF_IPSTATUS:OK

AT*M2M*WF_IPSTATUS=1

<- *M2M*WF_IPSTATUS:OK

3.3 SoftAP Commands

AT*M2M*WF_APSTART

Configures and immediately starts SoftAP operation.

Set Command:

AT*M2M*WF_APSTART=<channel> <ssid> [auth] [password] [hidden]

Response:

*M2M*WF_APSTART:OK

// or

*M2M*WF_APSTART:ERROR [error_code]

Parameters:

ssid: up to 32 characters; wrap in double quotes if it contains spaces

auth: 0-open; 1-wpa psk; 2-wpa2-psk;3-wpa2/wpa3 4-wpa3 5-wpa2 enterprise

password: 8–64 ASCII characters; wrap in double quotes if it contains spaces

hidden: 0-broadcast SSID (default); 1-hidden

reason: 0-not supported (also returned for auth=0 when the firmware is built without open-SoftAP support)

Example:

// open AP on channel 6

AT*M2M*WF_APSTART=6 m2m_ap

<- *M2M*WF_APSTART:OK

// WPA2-PSK & hidden

AT*M2M*WF_APSTART= 11 “m2m_ap” 3 12345678 1

<- *M2M*WF_APSTART:OK

AT*M2M*WF_APSTATION

Lists the stations currently connected to the SoftAP.

Execute Command:

AT*M2M*WF_APSTATION

Response:

*M2M*WF_APSTATION:IND <ip> <mac>   // one line per station

*M2M*WF_APSTATION:OK

// or

*M2M*WF_APSTATION:ERROR [error_code]

Parameters:

reason: 0-not supported

Example:

AT*M2M*WF_APSTATION

<- *M2M*WF_APSTATION:IND 192.168.1.2 00:01:02:03:04:05

<- *M2M*WF_APSTATION:IND 192.168.1.2 00:01:02:03:04:05

<- *M2M*WF_APSTATION:OK

3.4 WPS

AT*M2M*WF_WPS

Starts a WPS push-button session. Press the WPS button on the access point (physical or virtual) within the WPS walk time; the module completes the security setup and joins automatically.

Set Command:

AT*M2M*WF_WPS=<flag> [pin]

Response:

*M2M*WF_WPS:OK

// or

*M2M*WF_WPS:ERROR [error_code]

Parameters:

flag: 0-cancel; 1-start push-button mode, 2-start pin mode

pin: 8 decimal digits

reason: 0-not supported

Example:

AT*M2M*WF_WPS=1

<- *M2M*WF_WPS:OK

AT*M2M*WF_WPS=2 12345678

<- *M2M*WF_WPS:OK

3.5 BLE Provisioning

AT*M2M*BLE_PROV

Starts or stops the Bluetooth Low Energy (BLE) provisioning window used by an end-user's phone app (Espressif Unified/BLE Provisioning-compatible) to deliver Wi-Fi credentials directly to the module, bypassing the host -- see the companion M2M BT Provisioning Architecture Review for the transport/security design. On a successful provisioning the module also enables AT*M2M*WF_APMODE=1 automatically so the connection survives a reboot, since BLE provisioning is normally a one-time end-user setup step rather than a host-managed session; the provisioning window itself never persists across a reset -- it always starts closed (state=0) and must be reopened explicitly if needed again. The phone app must present this module's proof-of-possession (PoP) to open a session -- see AT*M2M*BLE_POP.

Query Command:

AT*M2M*BLE_PROV=?

Response:

*M2M*BLE_PROV:OK <state> [remaining_s]

Set Command:

AT*M2M*BLE_PROV=<state> [timeout_s]

Response:

*M2M*BLE_PROV:OK

// then, asynchronously:

*M2M*BLE_PROV:IND CONNECTED

*M2M*BLE_PROV:IND CRED_RECEIVED

*M2M*BLE_PROV:IND CRED_FAIL

*M2M*BLE_PROV:DONE

// or

*M2M*BLE_PROV:ERROR [error_code]

Parameters:

state: 0-provisioning window closed; 1-provisioning window open

timeout_s: optional, Set only; auto-close after this many seconds if no phone completes provisioning (0 or omitted: no timeout). Ignored when state=0

remaining_s: Query only, present only when state=1 and a timeout was set; seconds remaining before the window auto-closes

reason: 0-not supported; 1-window timed out without a phone completing provisioning (Wi-Fi credentials only -- MQTT/AWS/other server connections are not part of this session; see the companion Architecture Review, section 5.1)

Known limitation:

Only one provisioning session (state=1) is supported per boot. Once a session has ended -- BLE_PROV=0, a timeout, or a start failure -- AT*M2M*SYS_RST is required before another BLE_PROV=1 will succeed; a further attempt in the same boot fails with ERROR 0. See the companion Architecture Review, section 5.3.

Example:

AT*M2M*BLE_PROV=1 300

<- *M2M*BLE_PROV:OK

AT*M2M*BLE_PROV=?

<- *M2M*BLE_PROV:OK 1 214

AT*M2M*BLE_PROV=0

<- *M2M*BLE_PROV:OK

AT*M2M*BLE_POP

Reads or sets the proof-of-possession (PoP) that a phone must present to open an AT*M2M*BLE_PROV session (protocomm Security 1). It is unique per module: on first boot the module generates 12 characters from its hardware random number generator (alphabet 2-9 and A-Z without I, L and O) and stores them in NV memory. The host can read it to print it on a label or show it as a QR code, or replace it with a value provisioned in manufacturing. AT*M2M*SYS_FACTORY makes the next boot generate a new one.

Query Command:

AT*M2M*BLE_POP=?

Response:

*M2M*BLE_POP:OK <pop>

Set Command:

AT*M2M*BLE_POP=<pop>

Response:

*M2M*BLE_POP:OK

// or

*M2M*BLE_POP:ERROR [error_code]

Parameters:

pop: 8–32 printable ASCII characters without spaces

reason: 0-not supported; 1-invalid <pop> format; 2-a provisioning window is open (close it with AT*M2M*BLE_PROV=0 first)

Example:

AT*M2M*BLE_POP=?

<- *M2M*BLE_POP:OK ME7KNB5QAGYH

4. TCP/IP AT Commands

Up to four sockets can be open at the same time: identified by socket descriptors 0–3.

AT*M2M*NET_CONN returning OK only confirms that the command was accepted; completion of the actual TCP handshake is reported asynchronously by the *M2M*NET_CONN:DONE message, and a close by the peer arrives as *M2M*NET_CLI_CLOSED.

4.1 Command Mode

In the default command mode the host exchanges payload through AT*M2M*SEND and the M2M*RECV messages, Multiple sockets can be served concurrently in this mode.

4.2 Socket and Address Commands

AT*M2M*NET_STATUS

Reports the status of every open link.

Query Command:

AT*M2M*NET_STATUS=?

Response:

*M2M*NET_STATUS:IND <link_id> <role> <type> <remote_ip> <remote_port> [local_port]

*M2M*NET_STATUS:OK

// or

*M2M*NET_STATUS:ERROR [code]

Parameters:

link_id: 0~3

role: 0-client; 1-server

type: tcp;udp;ssl

reason: 0-not supported

Example:

AT*M2M*NET_STATUS=?

<- *M2M*NET_STATUS:IND 0 1 tcp 0.0.0.0 0 80

<- *M2M*NET_STATUS:IND 0 0 tcp 192.168.100.10 9000 34300

<- *M2M*NET_STATUS:OK

AT*M2M*NET_CONN

Opens a TCP, UDP or SSL connection on the given link, to either a literal IPv4 address or a hostname resolved via DNS.

Set Command:

AT*M2M*NET_CONN=<link_id> <type> <remote_host> <remote_port> <local_port> [keep_alive] [cert_name]

Response:

*M2M*NET_CONN:OK

// then, asynchronously:

*M2M*NET_CONN:DONE <link_id>

// or

*M2M*NET_CONN:ERROR [error_code]

Parameters:

link_id: 0~3

type: tcp;udp;ssl

remote_host: literal IPv4 address (e.g. 192.168.100.1) or a hostname to resolve via DNS (e.g. iot.example.com)

local_port: 0-auto; 1-listen_port

keep_alive: TCP/SSL keep-alive interval in seconds, 0-7200 (0 = disabled)

cert_name: client certificate file previously stored with AT*M2M*NET_HTTPDOWNLOAD, used for mutual TLS; meaningful only when <type> is ssl, omit to skip client-certificate authentication

reason: 0-not supported

Example:

AT*M2M*NET_CONN=0 tcp 192.168.100.1 9000 0 0

<- *M2M*NET_CONN:OK

// then, asynchronously:

<- *M2M*NET_CONN:DONE 0

AT*M2M*NET_CONN=0 udp 192.168.100.1 9000 9100 0

<- *M2M*NET_CONN:OK

// then, asynchronously:

<- *M2M*NET_CONN:DONE 0

AT*M2M*NET_CONN=0 tcp iot.example.com 8883 0 0

<- *M2M*NET_CONN:OK

// then, asynchronously:

<- *M2M*NET_CONN:DONE 0

AT*M2M*NET_DISCONN

Closes one link, or all links when the argument is omitted in single-connection mode

Set Command:

AT*M2M*NET_DISCONN=<link_id>

Response:

*M2M*NET_DISCONN:OK

// then, asynchronously:

*M2M*NET_DISCONN:DONE <link_id>

// or

*M2M*NET_DISCONN:ERROR [error_code]

Parameters:

link_id: 0~3

reason: 0-not supported

Example:

AT*M2M*NET_DISCONN=0

<- *M2M*NET_DISCONN:OK

<- *M2M*NET_DISCONN:DONE 0

AT*M2M*NET_DISCONN

<- *M2M*NET_DISCONN:OK

<- *M2M*NET_DISCONN:DONE 0

<- *M2M*NET_DISCONN:DONE 1

<- *M2M*NET_DISCONN:DONE 2

<- *M2M*NET_DISCONN:DONE 3

AT*M2M*NET_SEND

Sends payload on an open link

Set Command:

AT*M2M*NET_SEND=<link_id> <size> <payload>

Response:

*M2M*NET_SEND:OK

// or

*M2M*NET_SEND:ERROR [error_code]

Parameters:

link_id: 0~3

sizes: payload length, up to 8192 bytes

reason: 0-not supported

Example:

// as TCP server, to a specific client

AT*M2M*NET_SEND=1 5 Hello

<- *M2M*NET_SEND:OK

// as connected TCP client

AT*M2M*NET_SEND=0 5 Hello

<- *M2M*NET_SEND:OK

AT*M2M*NET_SERVER

Creates or removes a TCP/UDP/SSL server.

Query Command:

AT*M2M*NET_SERVER=?

Response:

*M2M*NET_SERVER:IND <type> <port>

*M2M*NET_SERVER:OK

Set Command:

AT*M2M*NET_SERVER=<mode> <type> <local_port> [cert_name]

Response:

*M2M*NET_SERVER:OK

// or

*M2M*NET_SERVER:ERROR [error_code]

Parameters:

mode: 0-disable server; 1-enable server

type: tcp;udp;ssl

cert_name: server certificate+key file previously stored with AT*M2M*NET_HTTPDOWNLOAD; required when <type> is ssl

reason: 0-not supported

Example:

AT*M2M*NET_SERVER=1 tcp 9000

<- *M2M*NET_SERVER:OK

AT*M2M*NET_PING

Sends ICMP echo requests. Each reply is reported with a *M2M*NET_PING:IND message, followed by *M2M*NET_PING:DONE.

Set Command:

AT*M2M*NET_PING=<count> <target_ip> <size>

Response:

*M2M*NET_PING:OK

// then, asynchronously (one line per reply):

*M2M*NET_PING:IND <ip> <size> <time_ms>

*M2M*NET_PING:DONE <sent> <received>

// or

*M2M*NET_PING:ERROR [error_code]

Parameters:

count: number of requests

size: payload size, up to 1452 bytes

reason: 0-not supported

Example:

AT*M2M*NET_PING=10 192.168.1.1 1460

<- *M2M*NET_PING:OK

// then, asynchronously:

<- *M2M*NET_PING:IND 192.168.1.1 1460 131

<- *M2M*NET_PING:DONE 10 10

AT*M2M*NET_DNS

Resolves a host name to an IP address.

Set Command:

AT*M2M*NET_DNS=<hostname>

Response:

*M2M*NET_DNS:OK <ip>

// or

*M2M*NET_DNS:ERROR [code]

Example:

AT*M2M*NET_DNS=www.example.com

<- *M2M*NET_DNS:OK 93.184.216.34

AT*M2M*NET_DTMODE

Selects normal or passthrough (transparent) transmission mode.

Query Command:

AT*M2M*NET_DTMODE=?

Response:

*M2M*NET_DTMODE:OK <mode> <link_id> <interval_ms>

Set Command:

AT*M2M*NET_DTMODE=<mode> <link_id> [interval_ms.

Response:

*M2M*NET_DTMODE:OK <mode> <link_id>

// or

*M2M*NET_DTMODE:ERROR [error_code]

Parameters:

mode: 0-normal (default); 1-passthrough:UART bytes are relayed to the single open link as-is

interval_ms: 10–1000 ms; out-of-range values fall back to the default (typically 200 ms)

reason: 0-not supported

Notes:

Leave passthrough mode with the escape sequence "+++" (three plus characters, no other characters, followed by at least 20 ms of silence on either side).

Example:

// TCP client

AT*M2M*NET_DTMODE=1 0

<- *M2M*NET_DTMODE:OK 1 0

// TCP server

AT*M2M*NET_DTMODE=1 0

<- *M2M*NET_DTMODE:OK 1 0

AT*M2M*NET_SNTP

Fetches the current GMT time. The module tries the configured NTP server first (default pool.ntp.org) and falls back to time.nist.gov and time.windows.com. The time string arrives as *M2M*NET_SNTP:IND, followed by *M2M*NET_SNTP:DONE.

Execute Command:

AT*M2M*NET_SNTP

Response:

*M2M*NET_SNTP:OK

// then, asynchronously:

*M2M*NET_SNTP:IND <length> <time_string>

*M2M*NET_SNTP:DONE

// or

*M2M*NET_SNTP:ERROR [error_code]

Parameters:

reason: 0-not supported, 1~: See Appendix C

Example:

AT*M2M*NET_SNTP

<- *M2M*NET_SNTP:OK

// then, asynchronously:

<- *M2M*NET_SNTP:IND 24 Thu Aug 22 11:45:48 2025

<- *M2M*NET_SNTP:DONE

AT*M2M*NET_SNTPCONF

Reads & Set one SNTP client property.

Query Command:

AT*M2M*NET_SNTPCONF=?

Response:

*M2M*NET_SNTPCONF:OK <mode> <offset> <url>

Example:

AT*M2M*SNTPCONF=?

<- *M2M*SNTPCONF:OK pool.ntp.org

Set Command:

AT*M2M*NET_SNTPCONF=<mode> [offset]

Response:

*M2M*NET_SNTPCONF:OK

// or

*M2M*NET_SNTPCONF:ERROR [error_code]

Parameters:

mode: 0-NTP server URL or IP address; 1-GMT offset

offset: time-zone offset from GMT

reason: 0-not supported, 1~: See Appendix C

Example:

AT*M2M*NET_SNTPCONF=0 time.windows.com

<- *M2M*NET_SNTPCONF:OK

AT*M2M*NET_SNTPCONF=1 9

<- *M2M*NET_SNTPCONF:OK

6. Network Service AT Commands

This chapter covers the built-in application clients and servers: DHCP, HTTP client and file download, embedded web server, an MQTT client, AWS IoT Core connectivity, and over-the-air upgrade.

6.1 DHCP Server / Client

AT*M2M*NET_DHCPS

Reads or sets the address range leased by the SoftAP DHCP server.

Query Command:

AT*M2M*NET_DHCPS=?

Response:

*M2M*NET_DHCPS:OK <lease_min> <start_ip> <end_ip>

// or

*M2M*NET_DHCPS:ERROR [error_code]

Set Command:

AT*M2M*NET_DHCPS=<flag> <lease_min> <start_ip> <end_ip>

Response:

*M2M*NET_DHCPS:OK

Parameters:

flag: 0-disable; 1-enable

lease time: lease time in minutes, 1-2880, 0 : system default value

reason: 0-not supported, 1~: See Appendix C

Example:

AT*M2M*NET_DHCPS=1 60 192.168.1.10 192.168.1.30

<- *M2M*NET_DHCPS:OK

AT*M2M*NET_DHCP

Enables or disables the DHCP client (station) and DHCP server (SoftAP)..

Query Command:

AT*M2M*NET_DHCP=? [mode]

Response:

*M2M*NET_DHCP:OK <mode> <flag>

// or

*M2M*NET_DHCP:ERROR [error_code]

Set Command:

AT*M2M*NET_DHCP=<mode> <flag>

Response:

*M2M*NET_DHCP:OK

Parameters:

mode: 0- station DHCP client, 1:SoftAP DHCP server

flag: 0-disable; 1-enable

reason: 0-not supported, 1~: See Appendix C

Example:

AT*M2M*NET_DHCP=1 1

<- *M2M*NET_DHCP:OK

6.2 HTTP Client

AT*M2M*NET_HTTPSET

Selects the Content-Type header sent with HTTP requests.

Set Command:

AT*M2M*NET_HTTPSET=<type> {scheme} [timeout] [response]

Response:

*M2M*NET_HTTPSET:OK

// or

*M2M*NET_HTTPSET:ERROR

Parameters:

type: 0-form, 1-json, 3-binary 4-text (0 : default)

scheme: 0-http; 1-https

timeout: 1-600

reason: 0-not supported, 1~:See Appendix C

Example:

AT*M2M*NET_HTTPSET=3 600

<- *M2M*NET_HTTPSET:OK

AT*M2M*NET_HTTPHEADER

Adds a custom header line to subsequent HTTP GET/POST requests.

Since the value persists across sessions, the header is reused even when the URL changes (i.e., on every request), It can be initialized using CLR when needed. Up to 5 headers can be maintained.

Set Command:

AT*M2M*NET_HTTPHEADER=<cmd> <key> <value>

Response:

*M2M*NET_HTTPHEADER:OK

// or

*M2M*NET_HTTPHEADER:ERROR [error_code]

Parameters:

cmd: ADD; CLR; DEL

key: header name (Content-Type, Authorization)

value : header values

reason: 0-not supported, 1~:-See Appendix C

Example:

AT*M2M*NET_HTTPHEADER=ADD "Authorization" "Bearer abcd1234"

<- *M2M*NET_HTTPHEADER:OK

AT*M2M*NET_HTTPHEADER=DEL "Authorization"

<- *M2M*NET_HTTPHEADER:OK

AT*M2M*NET_HTTPHEADER=CLR

<- *M2M*NET_HTTPHEADER:OK

AT*M2M*NET_HTTPGET

Issues an HTTP GET request. The response body is streamed back with *M2M*NET_HTTPGET:IND messages, followed by *M2M*NET_HTTPGET:DONE when the session ends.

Set Command:

AT*M2M*NET_HTTPGET=<url>[,<recv_mode>]

Response:

*M2M*NET_HTTPGET:OK

// then, asynchronously:

*M2M*NET_HTTPGET:IND <status_code> <chunk_len> <data>

*M2M*NET_HTTPGET:DONE

// or

*M2M*NET_HTTPGET:ERROR [error_code]

Parameters:

url: request target URL (up to 512 bytes),

recv_mode: 0-Immediately return the entire response body as a URC (default). 1-Notify only the response length, then retrieve the response separately using AT+HTTPREAD (optional) implementation for handling large responses

reason: 0-not supported, 1~:See Appendix C

Example:

AT*M2M*NET_HTTPGET=192.168.0.10/api/temp

<- *M2M*NET_HTTPGET:OK

-> *M2M*NET_HTTPGET:IND 200 27 {"temp":23.5,"unit":"C"}

-> *M2M*NET_HTTPGET:DONE

// HTTPS server (with scheme)

AT*M2M*NET_HTTPGET=https://192.168.0.10/api/temp

<- *M2M*NET_HTTPGET:OK

-> *M2M*NET_HTTPGET:IND 200 27 {"temp":23.5,"unit":"C"}

-> *M2M*NET_HTTPGET:DONE

// HTTP server (without scheme)

AT*M2M*HTTPSET=0 1

AT*M2M*NET_HTTPGET=192.168.0.10/api/temp

<- *M2M*NET_HTTPGET:OK

-> *M2M*NET_HTTPGET:IND 200 27 {"temp":23.5,"unit":"C"}

-> *M2M*NET_HTTPGET:DONE

AT*M2M*NET_HTTPPOST

Issues an HTTP POST request, optionally with a body. The response body is streamed back with *M2M*NET_HTTPPOST:IND messages, followed by *M2M*NET_HTTPPOST:DONE when the session ends.

Set Command:

AT*M2M*NET_HTTPPOST=<urL> [length] [data]

Response:

*M2M*NET_HTTPPOST:OK

// then, asynchronously:

*M2M*NET_HTTPPOST:IND <status_code> <data_len> <data>

*M2M*NET_HTTPPOST:DONE

// or

*M2M*NET_HTTPPOST:ERROR [error_code]

Parameters:

url: request target URL (up to 512 bytes),

length: length in bytes

data : The body encoding/interpretation method follows the data_type setting of AT+HTTPSET

reason: 0-not supported, 1~:See Appendix C

Example:

AT*M2M*NET_HTTPSET=1

AT*M2M*NET_HTTPPOST="https://api.example.com/v1/report" 21 {"status":"alive"}

-> *M2M*NET_HTTPPOST:OK

// then, asynchronously:

-> *M2M*NET_HTTPPOST:IND 201 15 {"result":"ok"}

-> *M2M*NET_HTTPPOST:DONE

AT*M2M*NET_HTTPSET=3

AT*M2M*NET_HTTPPOST="https://api.example.com/v1/report" 1024 <raw data>

-> *M2M*NET_HTTPPOST:OK

// then, asynchronously:

-> *M2M*NET_HTTPPOST:IND 201 2 OK

-> *M2M*NET_HTTPPOST:DONE

AT*M2M*NET_HTTPDOWNLOAD

Downloads a file over HTTP and stores it in the module file system — typically used to load certificate files for the SSL and EAP features. The final result arrives as a second line after the immediate acknowledgement.

Downloads are typically binary data, so they are always stored as raw bytes regardless of the data_type setting of AT+HTTPSET

The module file system has about 228 KB of usable space in total, shared by every stored file (certificates, keys and other downloads); use AT*M2M*NET_FILELIST to check what is already stored. A file holding a private key (a PEM "PRIVATE KEY" block, or a name ending in .key, .p12, .pfx or .pac) is kept in the module's encrypted parameter storage instead, and may be at most 8 KB.

Set Command:

AT*M2M*NET_HTTPDOWNLOAD=<url> <filename> [,<timeout_ms>]

Response:

*M2M*NET_HTTPDOWNLOAD:OK

// then, asynchronously:

*M2M*NET_HTTPDOWNLOAD:DONE

// or

*M2M*NET_HTTPDOWNLOAD:ERROR [error_code]

Parameters:

url: request target URL (up to 512 bytes),

filename: Internal file name, 1-30 characters of letters, digits, '.', '_' and '-', not starting with '.' (any other name is refused with reason 2)

reason: 0-not supported, 1~:See Appendix C, 5-the downloaded file holds a certificate or private key weaker than RSA-2048 / EC P-256, or is a private-key file larger than 8 KB (the file is not kept)

Example:

AT*M2M*NET_HTTPDOWNLOAD="https://fw.example.com/bk7239_v1.2.bin","fw_v1.2.bin",60000

*M2M*NET_HTTPDOWNLOAD:OK

<- *M2M*NET_HTTPDOWNLOAD:DONE

AT*M2M*NET_FILELIST

Lists every file currently stored on the module's file system (populated by AT*M2M*NET_HTTPDOWNLOAD or the configuration web server's certificate upload).

Execute Command:

AT*M2M*NET_FILELIST

Response:

*M2M*NET_FILELIST:IND <filename> <size>

*M2M*NET_FILELIST:OK

Parameters:

filename: internal file name

size: file size in bytes

Example:

AT*M2M*NET_FILELIST

<- *M2M*NET_FILELIST:IND ca.pem 1785

<- *M2M*NET_FILELIST:OK

AT*M2M*NET_FILEINFO

Reports the size of one stored file, and its SHA-256 fingerprint if the file is a PEM X.509 certificate.

Query Command:

AT*M2M*NET_FILEINFO=<filename>

Response:

*M2M*NET_FILEINFO:OK <filename> <size> [fingerprint]

// or

*M2M*NET_FILEINFO:ERROR [error_code]

Parameters:

filename: internal file name

size: file size in bytes

fingerprint: SHA-256 fingerprint, colon-separated uppercase hex; present only when the file is a PEM certificate

reason: 1-file not found

Example:

AT*M2M*NET_FILEINFO=ca.pem

<- *M2M*NET_FILEINFO:OK ca.pem 1785 14:05:37:D5:C0:1F:3F:E8:EC:54:D7:FB:39:64:82:0C:57:A3:38:40:46:F5:71:3E:28:FC:89:64:03:D7:95:4C

AT*M2M*NET_HTTPSTOP

Closes the current HTTP or HTTPS session.

Set Command:

AT*M2M*NET_HTTPSTOP

Response:

*M2M*NET_HTTPSTOP:OK

// session missing or still receiving:

*M2M*NET_HTTPSTOP:ERROR [error_code]

Parameters:

url: request target URL (up to 512 bytes),

filename: Internal file name

reason: 0-not supported, 1-no request to abort

Example:

AT*M2M*NET_HTTPSTOP

<- *M2M*NET_HTTPSTOP:OK

// then, asynchronously:

<- *M2M*NET_HTTPSTOP:DONE

6.3 HTTP Server

AT*M2M*NET_HTTPDSTART

Enables & Start configuration web server. The setting is stored in NV memory and survives a reboot.

Execute Command:

AT*M2M*NET_HTTPDSTART=<port> [,<ssl>] [,<cert_name>]

Response:

*M2M*NET_HTTPDSTART:OK

// or

*M2M*NET_HTTPDSTART:ERROR [error_code]

Parameters:

port: listen port

ssl: 0-plain HTTP (default); 1-HTTPS

cert_name: server certificate+key file previously stored with AT*M2M*NET_HTTPDOWNLOAD; required when <ssl> is 1

reason: 0-not supported, 1-port already in use; 2-SSL enabled but certificate/key not configured; 3-already running; 4-requested transport (plain HTTP or HTTPS) is disabled in this firmware build.

Example:

AT*M2M*NET_HTTPDSTART=80

<- *M2M*NET_HTTPDSTART:OK

AT*M2M*NET_HTTPDSTOP

Disables & Stop web server. The setting is stored in NV memory and survives a reboot.

Execute Command:

AT*M2M*NET_HTTPDSTOP

Response:

*M2M*NET_HTTPDSTOP:OK

// or

*M2M*NET_HTTPDSTOP:ERROR [error_code]

Parameters:

port: listen port

reason: 0-not supported, 1-no service

Example:

AT*M2M*NET_HTTPDSTOP

<- *M2M*NET_HTTPDSTOP:OK

AT*M2M*NET_HTTPDCONF

Reads or sets the login credentials of the configuration web server. The setting is stored in NV memory. Only a salted hash of the password is kept, so the Query form reports ******** in place of the password. The factory default is admin / admin; at the first web login it only allows setting a new password, and after 5 wrong passwords web login is locked for 1 minute, doubling on each further lockout up to 1 hour. A successful Set also clears any lockout and logs out the current web session.

Query Command:

AT*M2M*NET_HTTPDCONF=?

Response:

*M2M*NET_HTTPDCONF:OK <id> ********

Set Command:

AT*M2M*NET_HTTPDCONF=<id> <pw>

Response:

*M2M*NET_HTTPDCONF:OK

// or

*M2M*NET_HTTPDCONF:ERROR [error_code]

Parameters:

id: login name, 1–31 printable ASCII characters without spaces

password: 8–64 printable ASCII characters without spaces, containing at least one letter and one digit, and different from <id> and from the factory default

reason: 0-not supported; 1-id or password does not meet the rules above

Example:

AT*M2M*NET_HTTPDCONF=?

<- *M2M*NET_HTTPDCONF:OK admin ********

AT*M2M*NET_HTTPDCONF=admin Config2026

<- *M2M*NET_HTTPDCONF:OK

6.4 MQTT Client

AT*M2M*MQTT_CONF

Configures one MQTT client profile by index: broker address, transport scheme and client identity. Call this before AT*M2M*MQTT_CONN.

Query Command:

AT*M2M*MQTT_CONF=?

Response:

*M2M*MQTT_CONF:OK <scheme> <host> <port> [path] <client_id> [user_name] [cert_name]

Set Command:

AT*M2M*MQTT_CONF=<link_id> <scheme> <host> <port> [path] <client_id> [user_name] [password] [cert_name]

Response:

*M2M*MQTT_CONF:OK

// or

*M2M*MQTT_CONF:ERROR [error_code]

Parameters:

link_id: 0-3

scheme: 0: mqtt (plain TCP); 1: mqtts (TLS); 2: ws; 3: wss

host : broker address, domain name or IP

path : WebSocket path (e.g. /mqtt); meaningful only when <scheme> is ws or wss

client_id : up to 64 characters

user_name : MQTT-level authentication, optional

password: MQTT-level authentication, optional

cert_name:  certificate file name previously stored with AT*M2M*NET_HTTPDOWNLOAD; required when <scheme> is mqtts or wss and the broker needs a client certificate

reason: 0-not supported

Example:

AT*M2M*MQTT_CONF=0 0 "iot.example.com" 1883 "" "device01" "" "" ""

<- *M2M*MQTT_CONF:OK

AT*M2M*MQTT_ALPN

Sets the ALPN protocol list offered in the TLS handshake -- required by some brokers (including AWS IoT) that multiplex MQTT and HTTPS on port 443

Set Command:

AT*M2M*MQTT_ALPN=<link_id> <count> [alpn1] [alpn2] ...

Response:

*M2M*MQTT_ALPN:OK

Parameters:

count: 0-5; 0 clears the configured list

reason: 0-not supported

Example:

AT*M2M*MQTT_ALPN=0 1 "x-amzn-mqtt-ca"

<- *M2M*MQTT_ALPN:OK

AT*M2M*MQTT_CONN

Connects to the broker using the profile set for <link_id> with AT*M2M*MQTT_CONF

Set Command:

AT*M2M*MQTT_CONN=<link_id> [keep_alive]

Response:

*M2M*MQTT_CONN:OK

// then, asynchronously:

*M2M*MQTT_CONN:IND <link_id> <scheme> <host> <port> <path> <reconnect>

*M2M*MQTT_CONN:DONE

// or

*M2M*MQTT_CONN:ERROR [error_code]

Parameters:

Keep_alive: MQTT keep-alive interval in seconds, 0-7200 (0 = disabled), default 120

reconnect: 0: the module will not retry automatically after a link loss; 1: automatic reconnect is enabled

reason: 0-not supported, 1-<link_id> not configured -- run AT*M2M*MQTT_CONF first; 2: could not reach the broker; 3: TLS handshake failed; 4: broker rejected the connection (bad credentials); 5: already connected

Example:

AT*M2M*MQTT_CONN=0

<- *M2M*MQTT_CONN:OK

// then, asynchronously:

<- *M2M*MQTT_CONN:IND 0 0 iot.example.com 1883  1

<- *M2M*MQTT_CONN:DONE

AT*M2M*MQTT_STATUS

Reports the live connection state of every configured MQTT link. AT*M2M*MQTT_CONF's own query only reports static configuration, never whether the link is actually connected right now -- that was previously only observable via the asynchronous *M2M*MQTT_CONN:IND / *M2M*MQTT_DISCONN:IND notifications, which a host that just booted or missed one has no way to recover.

Query Command:

AT*M2M*MQTT_STATUS=?

Response:

*M2M*MQTT_STATUS:IND <link_id> <configured> <connected> <scheme> <host> <port>

*M2M*MQTT_STATUS:OK

Parameters:

link_id: 0-3

configured: 0-not configured; 1-configured

connected: 0-disconnected; 1-connected

scheme: 0-mqtt (plain TCP); 1-mqtts (TLS); 2-ws; 3-wss

Example:

AT*M2M*MQTT_STATUS=?

<- *M2M*MQTT_STATUS:IND 0 1 1 0 192.168.10.2 1883

<- *M2M*MQTT_STATUS:OK

AT*M2M*MQTT_PUB

Publishes a short string payload to a topic.

Set Command:

AT*M2M*MQTT_PUB=<link_id> <topic> <data> <qos> <retain> [timeout_ms]

Response:

*M2M*MQTT_PUB:OK

// or

*M2M*MQTT_PUB:ERROR [error_code]

Parameters:

topic: up to 128 bytes

data: string payload, must not contain the null byte

qos: 0-2 (default 0)

retain: 0 or 1

Example:

AT*M2M*MQTT_PUB=0 "device01/status" "{""power"":""on""}" 0 0

<- *M2M*MQTT_PUB:OK

AT*M2M*MQTT_SUB

Reads the current subscriptions, or subscribes to a topic (up to 10 topics at once)

Query Command:

AT*M2M*MQTT_SUB=?

Response:

*M2M*MQTT_SUB:IND <link_id> <state> <topic> <qos>   // one line per subscription

*M2M*MQTT_SUB:OK

// or

*M2M*MQTT_SUB:ERROR [error_code]

Set Command:

AT*M2M*MQTT_SUB=<cmd> <link_id> <topic> [qos] [timeout_ms]

Response:

*M2M*MQTT_SUB:OK

// or

*M2M*MQTT_SUB:ERROR [error_code]

Parameters:

cmd: 0: subscribe; 1:unsubscribe

topic: up to 128 bytes

qos: 0-2 (default 0)

timeout:

reason: 0-not supported; 1-no subscribe 2-already subscribe

Example:

AT*M2M*MQTT_SUB=0 0 "device01/cmd" 1

<- *M2M*MQTT_SUB:OK

AT*M2M*MQTT_SUB=1 0 "device01/cmd" 1

<- *M2M*MQTT_SUB:OK

AT*M2M*MQTT_CLEAN

Disconnects and releases the MQTT connection resources.

Set Command:

AT*M2M*MQTT_CLEAN=<link_id>

Response:

*M2M*MQTT_CLEAN:OK

// then, asynchronously

*M2M*MQTT_CLEAN:DONE

// or

*M2M*MQTT_CLEAN:ERROR [error_code]

Example:

AT*M2M*MQTT_CLEAN=0

<- *M2M*MQTT_CLEAN:OK

<- *M2M*MQTT_CLEAN:DONE

6.5 AWS Client

The commands below automate AWS IoT Core provisioning and connectivity on top of the MQTT client above. AT*M2M*AWS_PAIR exchanges a one-time activation code for the location of a certificate-provisioning service; AT*M2M*AWS_CERT and AT*M2M*AWS_CONN fetch and use the X.509 certificate from that service on demand, so the certificate itself never has to be programmed at manufacturing time and can be re-issued without repeating the pairing step. For fleets that use AWS IoT's own Fleet Provisioning instead of a custom pairing service, AT*M2M*AWS_CLAIMCERT and AT*M2M*AWS_PROVISION exchange a pre-installed claim certificate for a permanent one directly against AWS IoT's certificate and provisioning-template topics, feeding the same AT*M2M*AWS_CERT / AT*M2M*AWS_CONN certificate store.

AT*M2M*AWS_PAIR

Pairs the module with a provisioning service using a one-time activation code and learns where to fetch its AWS IoT certificate. Run once after manufacturing, or again after AT*M2M*AWS_PAIR=RESET.

Note (deprecated): this custom pairing/provisioning path has no defined wire protocol for the pairing or certificate server (never specified anywhere in this document), so it cannot be implemented against a real service. New integrations should use AWS IoT's standard Fleet Provisioning by Claim instead -- AT*M2M*AWS_CLAIMCERT + AT*M2M*AWS_PROVISION, fully specified and verified against real AWS IoT infrastructure.

Query Command:

AT*M2M*AWS_PAIR=?

Response:

*M2M*AWS_PAIR:OK <state> "<cert_server_url>"

Set Command:

AT*M2M*AWS_PAIR=<pairing_code> [pairing_server_url]

// or, to clear a previous pairing:

AT*M2M*AWS_PAIR=RESET

Response:

*M2M*AWS_PAIR:OK

// then, asynchronously:

*M2M*AWS_PAIR:DONE "<cert_server_url>"

// or

*M2M*AWS_PAIR:ERROR [error_code]

Parameters:

- <pairing_code>: one-time or short-lived activation code issued for this device (delivered out of band, e.g. QR, NFC or BLE)

- [pairing_server_url]: overrides the pairing server built into the firmware; intended for region/environment testing only

- <state>: 0: not paired; 1: paired (a certificate may or may not be present yet, see AT*M2M*AWS_CERT)

- <reason>: 1: malformed pairing_code; 2: could not reach the pairing server; 3: pairing_code rejected (expired, already used, or invalid); 4: already paired -- run AT*M2M*AWS_PAIR=RESET first

Example:

AT*M2M*AWS_PAIR="AB12-CD34-EF56"

<- *M2M*AWS_PAIR:OK

<- *M2M*AWS_PAIR:DONE "https://provision.example.com/v1/device"

AT*M2M*AWS_CERT

Reads the AWS IoT certificate status, or forces the module to fetch or discard it.

Note (deprecated for op=FETCH only): Query and DEL work regardless of provisioning path and are the recommended way to check status after AT*M2M*AWS_PROVISION. op=FETCH belongs to the deprecated AT*M2M*AWS_PAIR pairing flow (see that command's note) and always reports "not paired" in this implementation.

Query Command:

AT*M2M*AWS_CERT=?

Response:

*M2M*AWS_CERT:OK <state> "<thing_name>" "<endpoint>" <expiry_epoch>

Set Command:

AT*M2M*AWS_CERT=<op>

Response:

*M2M*AWS_CERT:OK

// then, asynchronously (op=FETCH only):

*M2M*AWS_CERT:IND FETCHING

*M2M*AWS_CERT:DONE "<thing_name>" "<endpoint>"

// or

*M2M*AWS_CERT:ERROR [error_code]

Parameters:

- <op>: FETCH: force a fresh certificate to be issued; DEL: discard the local certificate (pairing information is kept, the next AT*M2M*AWS_CONN fetches a new one automatically)

- <state>: 0: no certificate present; 1: certificate present; 2: certificate expiring soon (renewal recommended)

- <reason>: 1: not paired yet -- run AT*M2M*AWS_PAIR first; 2: could not reach the certificate server; 3: rejected by the certificate server -- re-pair; 4: malformed server response; 5: local storage error

Example:

AT*M2M*AWS_CERT=?

<- *M2M*AWS_CERT:OK 0 "" "" 0

AT*M2M*AWS_CERT=FETCH

<- *M2M*AWS_CERT:OK

<- *M2M*AWS_CERT:IND FETCHING

<- *M2M*AWS_CERT:DONE "device-0123" "a1b2c3-ats.iot.ap-northeast-2.amazonaws.com"

AT*M2M*AWS_CLAIMCERT

Configures the certificate files used as the AWS IoT claim certificate for AT*M2M*AWS_PROVISION. The referenced files must already be stored on the module.

Query Command:

AT*M2M*AWS_CLAIMCERT=?

Response:

*M2M*AWS_CLAIMCERT:OK "<ca_file>" "<cert_file>" "<key_file>"

Set Command:

AT*M2M*AWS_CLAIMCERT=<ca_file> <cert_file> <key_file>

Response:

*M2M*AWS_CLAIMCERT:OK

// or

*M2M*AWS_CLAIMCERT:ERROR [error_code]

Parameters:

- <ca_file>: root CA certificate file name (max. 16 characters) for the claim certificate chain

- <cert_file>: claim certificate file name (max. 16 characters)

- <key_file>: claim private-key file name (max. 16 characters)

- <reason>: 1: referenced file not found on the module; 2: malformed certificate or key

Example:

AT*M2M*AWS_CLAIMCERT=ca.pem claim.pem claim-key.pem

<- *M2M*AWS_CLAIMCERT:OK

AT*M2M*AWS_PROVISION

Runs AWS IoT Fleet Provisioning by Claim on <link_id>: connects using the claim certificate configured with AT*M2M*AWS_CLAIMCERT, requests a new certificate ($aws/certificates/create), then registers the device against <template_name> ($aws/provisioning-templates/<template_name>/provision). On success the issued certificate becomes the active certificate reported by AT*M2M*AWS_CERT and the link stays connected under the permanent identity.

Query Command:

AT*M2M*AWS_PROVISION=?

Response:

*M2M*AWS_PROVISION:OK <state>

Set Command:

AT*M2M*AWS_PROVISION=<link_id> <template_name> [parameters_json]

Response:

*M2M*AWS_PROVISION:OK

// then, asynchronously:

*M2M*AWS_PROVISION:IND CONNECTING <link_id>

*M2M*AWS_PROVISION:IND REQUESTING_CERT <link_id>

*M2M*AWS_PROVISION:IND REGISTERING <link_id>

*M2M*AWS_PROVISION:DONE <link_id> "<thing_name>"

// or

*M2M*AWS_PROVISION:ERROR <link_id> [error_code]

Parameters:

- <link_id>: 0-3, shared with the other AWS/MQTT client commands

- <template_name>: name of the AWS IoT provisioning template authorized to accept this claim certificate

- [parameters_json]: optional JSON object of template parameters (e.g. serial number), forwarded as-is to the provisioning template

- <state>: 0: not provisioned; 1: provisioning in progress; 2: provisioned (a certificate is present, see AT*M2M*AWS_CERT)

- <reason>: 1: AT*M2M*AWS_CLAIMCERT not configured; 2: TLS handshake with the claim certificate failed; 3: certificate request rejected; 4: <template_name> rejected or parameters invalid; 5: <link_id> already connected

Example:

AT*M2M*AWS_PROVISION=0 "esp-fleet-template" {"SerialNumber":"SN-0123"}

<- *M2M*AWS_PROVISION:OK

<- *M2M*AWS_PROVISION:IND CONNECTING 0

<- *M2M*AWS_PROVISION:IND REQUESTING_CERT 0

<- *M2M*AWS_PROVISION:IND REGISTERING 0

<- *M2M*AWS_PROVISION:DONE 0 "device-0123"

AT*M2M*AWS_CONN

Connects to AWS IoT Core over MQTT/TLS on <link_id>, fetching the certificate automatically first if AT*M2M*AWS_CERT reports none present.

Set Command:

AT*M2M*AWS_CONN=<link_id> [port_mode]

Response:

*M2M*AWS_CONN:OK

// certificate missing, fetched automatically:

*M2M*AWS_CONN:IND PROVISIONING <link_id>

// then:

*M2M*AWS_CONN:DONE <link_id>

// or

*M2M*AWS_CONN:ERROR <link_id> [error_code]

Parameters:

- <link_id>: 0-3, shared with the MQTT client commands above

- [port_mode]: 0 (default): port 8883, mTLS, no ALPN; 1: port 443, mTLS with ALPN "x-amzn-mqtt-ca" (use when 8883 is blocked)

- <reason>: 1-5: same as AT*M2M*AWS_CERT; 10: TLS handshake failed; 11: MQTT CONNACK rejected; 12: <link_id> already connected

Example:

AT*M2M*AWS_CONN=0

<- *M2M*AWS_CONN:OK

<- *M2M*AWS_CONN:IND PROVISIONING 0

<- *M2M*AWS_CONN:DONE 0

AT*M2M*AWS_DISC

Disconnects an AWS IoT Core link opened with AT*M2M*AWS_CONN.

Set Command:

AT*M2M*AWS_DISC=<link_id>

Response:

*M2M*AWS_DISC:OK

// then, asynchronously:

*M2M*AWS_DISC:DONE <link_id>

// or

*M2M*AWS_DISC:ERROR [error_code]

Parameters:

- <reason>: 1: <link_id> not connected

Example:

AT*M2M*AWS_DISC=0

<- *M2M*AWS_DISC:OK

<- *M2M*AWS_DISC:DONE 0

AT*M2M*AWS_PUB

Publishes a payload to an AWS IoT Core topic on a connected link. Reserved topics such as $aws/things/<thing_name>/... may be addressed directly.

Set Command:

AT*M2M*AWS_PUB=<link_id> <topic> <qos> <retain> <data_len>

> <raw_payload>

Response:

*M2M*AWS_PUB:OK   // ready for payload, echoed by ">"

// after the payload is written, when qos>=1:

*M2M*AWS_PUB:DONE <link_id>

// or

*M2M*AWS_PUB:ERROR [error_code]

Parameters:

- <qos>: 0-1 (AWS IoT Core does not support QoS 2)

- <retain>: must be 0 (AWS IoT Core does not support retained messages)

- <data_len>: payload length in bytes, subject to the AWS IoT Core message-size limit (128 KB by default, account-dependent)

- <reason>: 1: not connected; 2: qos=2 or retain=1 requested; 3: <data_len> too large

Example:

AT*M2M*AWS_PUB=0 "device-0123/telemetry" 1 0 21

> {"temp":23.5,"hum":41}

<- *M2M*AWS_PUB:OK

<- *M2M*AWS_PUB:DONE 0

AT*M2M*AWS_SUB

Reads the current AWS IoT Core subscriptions, subscribes to a topic, or unsubscribes.

Query Command:

AT*M2M*AWS_SUB=?<link_id>

Response:

*M2M*AWS_SUB:IND <link_id> <topic> <qos>   // one line per subscription

*M2M*AWS_SUB:OK

Set Command:

AT*M2M*AWS_SUB=<link_id> <topic> <qos>

// or, to unsubscribe:

AT*M2M*AWS_SUB=DEL <link_id> <topic>

Response:

*M2M*AWS_SUB:OK

// or

*M2M*AWS_SUB:ERROR [error_code]

Parameters:

- <reason>: 1: not connected; 2: not subscribed to <topic> (DEL only)

Example:

AT*M2M*AWS_SUB=0 "device-0123/cmd" 1

<- *M2M*AWS_SUB:OK

<- *M2M*AWS_MSG:DONE 0 "device-0123/cmd" 13

{"led":"on"}

6.6 Firmware OTA

AT*M2M*OTA_CHECK

Compares the local firmware version against the image published on an OTA server

Set Command:

AT*M2M*OTA_CHECK=<url>

Response:

*M2M*OTA_CHECK:OK

// then, asynchronously:

*M2M*OTA_CHECK:DONE <local_version> <server_version>

// or

*M2M*OTA_CHECK:ERROR [error_code]

Parameters:

- <url>: base URL of the OTA image, http://

- <local_version>, <server_version>: software versions in the AT*M2M*SYS_VER <version> format (<MM>.<NN>). Compared as numbers (MM, then NN); the update is offered only when <server_version> is greater

- reason: 1: initialization error; 2: argument error

Example:

AT*M2M*OTA_CHECK=http://ota.example.com:8080/ota/

<- *M2M*OTA_CHECK:OK

<- *M2M*OTA_CHECK:DONE 01.00 01.01

AT*M2M*OTA_UPDATE

Downloads and installs the firmware image from the OTA server. Run AT*M2M*OTA_CHECK first; the request is refused when the server image is not newer. The final result arrives as *M2M*OTA_UPDATE.

Set Command:

AT*M2M*OTA_UPDATE=<url>

Response:

*M2M*OTA_UPDATE:OK

// or

*M2M*OTA_UPDATE:ERROR [error_code]

Parameters:

- <url>: base URL of the OTA image, http://

- reason: 1: initialization error; 2: argument error; 3: server version not newer than local version; 4: run AT*M2M*OTA_CHECK first; 5: downloaded image ID (hardware, AT command set, customer code) does not match the installed image

Example:

AT*M2M*OTA_UPDATE=http://ota.example.com:8080/ota/

<- *M2M*OTA_UPDATE:OK

<- *M2M*OTA_UPDATE:DONE

7. Notification Messages

Notification messages are sent by the module asynchronously; the host must be prepared to receive them at any time between command/response exchanges.

7.1 Message Summary

| Message | Meaning |
| --- | --- |
| *M2M*DEVICEREADY | Module has booted and is ready for commands |
| *M2M*WF_CONN:DONE ¦ ERROR [code] | Station association result (§3.2 AT*M2M*WF_CONN). |
| *M2M*WF_DISCONN:DONE <reason> | Station lost its AP association without a AT*M2M*WF_DISCONN request. |
| *M2M*STA_ASSOCIATED:<mac> <rssi> | A station joined the SoftAP (§3.3 AT*M2M*WF_APSTART). |
| *M2M*STA_DISASSOCIATED:<mac> | A station disjoined the SoftAP (§3.3 AT*M2M*WF_APSTART). |
| *M2M*NET_CONN:DONE <link_id> ¦ ERROR [code] | TCP/UDP/SSL connection result (§4.2 AT*M2M*NET_CONN). |
| *M2M*NET_DISCONN:DONE <link_id> | Link closed by request (§4.2 AT*M2M*NET_DISCONN). |
| *M2M*NET_IP:IND <ip> <subnet> <gateway> <dns> | IP address obtained (DHCP lease). |
| *M2M*NET_CLI_CLOSED:<link_id> [ip] [port] | Link ended from the remote side (peer closed, or a server-side client left). |
| *M2M*NET_RECV:IND <link_id> <ip> <port> <size> <data> | TCP/UDP/SSL payload received. |
| *M2M*NET_PING:IND ... ¦ DONE <sent> <received> ¦ ERROR [code] | Ping replies and summary (§4.2 AT*M2M*NET_PING). |
| *M2M*NET_SNTP:IND ... ¦ DONE ¦ ERROR [code] | SNTP time result (§4.2 AT*M2M*NET_SNTP). |
| *M2M*NET_HTTPGET:IND ... ¦ DONE ¦ ERROR [code] | HTTP GET response body (§6.2 AT*M2M*NET_HTTPGET). |
| *M2M*NET_HTTPPOST:IND ... ¦ DONE ¦ ERROR [code] | HTTP POST response (§6.2 AT*M2M*NET_HTTPPOST). |
| *M2M*NET_HTTPDOWNLOAD:DONE ¦ ERROR [code] | File download result (§6.2 AT*M2M*NET_HTTPDOWNLOAD). |
| *M2M*NET_HTTPSTOP:DONE | A pending HTTP session finished aborting (§6.2 AT*M2M*NET_HTTPSTOP). |
| *M2M*MQTT_CONN:IND ... ¦ DONE ¦ ERROR [code] | MQTT broker connection result (§6.4 AT*M2M*MQTT_CONN). |
| *M2M*BLE_PROV:IND CONNECTED ¦ CRED_RECEIVED ¦ CRED_FAIL ¦ DONE ¦ ERROR [code] | BLE provisioning progress (§3.5 AT*M2M*BLE_PROV). |
| *M2M*MQTT_DISCONN:IND<link_id> | MQTT broker link lost unexpectedly. |
| *M2M*MQTT_RECV:IND <link_id> <topic> <length> <data> | Message received on a subscribed MQTT topic. |
| *M2M*MQTT_CLEAN:DONE ¦ ERROR [code] | MQTT disconnect result (§6.4 AT*M2M*MQTT_CLEAN). |
| *M2M*AWS_PAIR:DONE "<url>" ¦ ERROR [code] | Provisioning pairing result (§6.5 AT*M2M*AWS_PAIR). |
| *M2M*AWS_CERT:IND FETCHING ¦ DONE ... ¦ ERROR [code] | Certificate fetch progress and result (§6.5 AT*M2M*AWS_CERT). |
| *M2M*AWS_PROVISION:IND CONNECTING ¦ REQUESTING_CERT ¦ REGISTERING <link_id> ¦ DONE <link_id> "<thing_name>" ¦ ERROR <link_id> [code] | Fleet Provisioning by Claim result (§6.5 AT*M2M*AWS_PROVISION). |
| *M2M*AWS_CONN:IND PROVISIONING <link_id> ¦ DONE <link_id> ¦ ERROR <link_id> [code] | AWS IoT Core connection result (§6.5 AT*M2M*AWS_CONN). |
| *M2M*AWS_DISC:DONE <link_id> ¦ ERROR [code] | AWS IoT Core disconnection result (§6.5 AT*M2M*AWS_DISC). |
| *M2M*AWS_PUB:DONE <link_id> | Publish acknowledged, QoS ≥ 1 (§6.5 AT*M2M*AWS_PUB). |
| *M2M*AWS_MSG:DONE <link_id> <topic> <data_len> | Message received on a subscribed AWS IoT Core topic. |
| *M2M*OTA_CHECK:DONE <local_version> <server_version> ¦ ERROR [code] | Firmware version comparison result (§6.6 AT*M2M*OTA_CHECK). |
| *M2M*OTA_UPDATE:DONE ¦ ERROR [code] | OTA installation result (§6.6 AT*M2M*OTA_UPDATE). |

7.2 Message Details

Messages that carry parameters are described below. Notification messages that complete a specific host command (the :IND / :DONE / :ERROR forms) are documented next to that command in Chapters 2–6; only messages the module can send without a preceding command are detailed here.

*M2M*WF_DISCONN

Reports that the station lost its connection to the AP without a host-initiated AT*M2M*WF_DISCONN.

Message Command:

*M2M*WF_DISCONN:DONE <reason>

Parameters:

- <reason>: 0: AP-initiated deauthentication; 1: beacon loss or out of range; 2: authentication or key-exchange failure

- Not sent for authentication failures that are being retried automatically (see AT*M2M*WF_CONN); only the final failure is reported.

Example:

<- *M2M*WF_DISCONN:DONE 1

*M2M*STA_ASSOCIATED

Announces a station that joined the SoftAP.

Message Command:

*M2M*STA_ASSOCIATED:<mac> <rssi>

Parameters:

- <mac>: station MAC address

- <rssi>: signal strength of the station

Example:

<- *M2M*STA_ASSOCIATED:DC:A9:71:37:53:A2 -44

*M2M*NET_IP

Announces the IP configuration obtained by the module, typically right after *M2M*WF_CONN:DONE.

Message Command:

*M2M*NET_IP:IND <ip> <subnet> <gateway> <dns>

Example:

<- *M2M*NET_IP:IND 192.168.100.100 255.255.255.0 192.168.100.1 164.124.101.2

*M2M*NET_CLI_ACCEPTED / *M2M*NET_CLI_CLOSED

Announce a remote client joining a TCP/SSL server opened with AT*M2M*NET_SERVER, and a link ending from the remote side — a server-side client leaving, or the peer of a client connection closing it.

Message Command:

*M2M*NET_CLI_ACCEPTED:<link_id> <ip> <port>

*M2M*NET_CLI_CLOSED:<link_id> [ip] [port]

Example:

<- *M2M*NET_CLI_ACCEPTED:1 192.168.0.64 9100

<- *M2M*NET_CLI_CLOSED:1 192.168.0.64 9100

*M2M*NET_RECV

Deliver received TCP/SSL payload and UDP datagrams respectively.

Message Command:

*M2M*NET_RECV:IND <link_id> <ip> <port> <size> <data>

*M2M*NET_RECV:IND <link_id> <ip> <port> <size> <data>

Example:

<- *M2M*NET_RECV:IND 0 192.168.0.64 9100 10 abcdef1234

<- *M2M*NET_RECV:IND 0 192.168.0.103 56362 10 abcdef1234

*M2M*MQTT_DISCONN

Reports that an MQTT broker link opened with AT*M2M*MQTT_CONN was lost without a host-initiated AT*M2M*MQTT_CLEAN.

Message Command:

*M2M*MQTT_DISCONN:IND <link_id>

Example:

<- *M2M*MQTT_DISCONN:IND 0

*M2M*MQTT_RECV

Delivers a message received on a topic subscribed with AT*M2M*MQTT_SUB.

Message Command:

*M2M*MQTT_RECV:IND <link_id> <topic> <length> <data>

Example:

<- *M2M*MQTT_RECV:IND 0 device01/cmd 16 {"power":"off"}

*M2M*AWS_MSG

Delivers a message received on an AWS IoT Core topic subscribed with AT*M2M*AWS_SUB. The payload follows on the next line.

Message Command:

*M2M*AWS_MSG:DONE <link_id> <topic> <data_len>

Example:

<- *M2M*AWS_MSG:DONE 0 "device-0123/cmd" 13

<- {"led":"on"}

8. Usage Examples

The listings below show complete command sequences for common tasks. Lines beginning with "//" are comments; responses and notification messages from the module are shown indented with "<-".

8.1 Joining an Access Point

The sequences below show the common connection variants: open network, WPA2-PSK, pinning to a specific BSSID, and WPS.

// --- open network ---

AT*M2M*WF_MODE=1

<- *M2M*WF_MODE:OK

AT*M2M*WF_CONN=office_ap

<- *M2M*WF_CONN:OK

// then, asynchronously:

<- *M2M*WF_CONN:DONE

<- *M2M*WF_CONN:IND 192.168.0.68 255.255.255.0 192.168.0.1 164.124.101.2

// --- WPA2-PSK, passphrase with a space ---

AT*M2M*WF_CONN=office_ap "1234 5678"

<- *M2M*WF_CONN:OK

<- *M2M*WF_CONN:DONE

<- *M2M*WF_CONN:IND 192.168.0.68 255.255.255.0 192.168.0.1 164.124.101.2

// --- pin to a specific BSSID ---

AT*M2M*WF_CONN=office_ap 12345678 00:01:02:03:04:05

<- *M2M*WF_CONN:OK

<- *M2M*WF_CONN:DONE

// --- WPS push button ---

// press the WPS button on the AP, then:

AT*M2M*WF_WPS=1

<- *M2M*WF_WPS:OK

8.2 Running a SoftAP

// --- open AP on channel 6 ---

AT*M2M*WF_MODE=2

<- *M2M*WF_MODE:OK

AT*M2M*WF_APSTART=6 m2m_ap

<- *M2M*WF_APSTART:OK

<- *M2M*STA_ASSOCIATED:DC:A9:71:37:53:A2 -44

// --- WPA2-PSK & hidden, channel 11 ---

AT*M2M*WF_APSTART=11 "m2m_ap" 3 12345678 1

<- *M2M*WF_APSTART:OK

AT*M2M*WF_APSTATION

<- *M2M*WF_APSTATION:IND 192.168.1.2 00:01:02:03:04:05

<- *M2M*WF_APSTATION:OK

8.3 TCP and UDP Sockets

// --- TCP client ---

AT*M2M*NET_CONN=0 tcp 192.168.0.64 9100 0 0

<- *M2M*NET_CONN:OK

<- *M2M*NET_CONN:DONE 0

AT*M2M*NET_SEND=0 5 Hello

<- *M2M*NET_SEND:OK

<- *M2M*NET_RECV:0 192.168.0.64 9100 5 World

AT*M2M*NET_DISCONN=0

<- *M2M*NET_DISCONN:OK

<- *M2M*NET_DISCONN:DONE 0

// --- TCP server ---

AT*M2M*NET_SERVER=1 tcp 9000

<- *M2M*NET_SERVER:OK

<- *M2M*NET_CLI_ACCEPTED:1 192.168.0.111 1691

<- *M2M*NET_RECV:1 192.168.0.111 1691 5 Hello

AT*M2M*NET_SEND=1 5 World

<- *M2M*NET_SEND:OK

// --- UDP client ---

AT*M2M*NET_CONN=2 udp 192.168.0.127 1000 9100 0

<- *M2M*NET_CONN:OK

<- *M2M*NET_CONN:DONE 2

AT*M2M*NET_SEND=2 5 Hello

<- *M2M*NET_SEND:OK

<- *M2M*NET_RECV:2 192.168.0.127 1000 5 World

// --- UDP server (server socket assigned link_id 3) ---

AT*M2M*NET_SERVER=1 udp 5000

<- *M2M*NET_SERVER:OK

<- *M2M*NET_RECV:3 192.168.0.103 56362 5 Hello

AT*M2M*NET_SEND=3 5 World

<- *M2M*NET_SEND:OK

8.4 TLS Connections

SSL/TLS is selected with the <type> parameter of AT*M2M*NET_CONN and AT*M2M*NET_SERVER (§4.2); the notification flow is identical to the plain TCP case. A certificate is supplied by name with the optional cert_name parameter, referencing a file already stored with AT*M2M*NET_HTTPDOWNLOAD (§6.2) -- the same mechanism the MQTT client uses (§6.4).

// --- TLS client, server-only validation (no client certificate) ---

AT*M2M*NET_CONN=0 ssl 192.168.0.10 8443 0 0

<- *M2M*NET_CONN:OK

<- *M2M*NET_CONN:DONE 0

AT*M2M*NET_SEND=0 5 Hello

<- *M2M*NET_SEND:OK

<- *M2M*NET_RECV:0 192.168.0.10 8443 5 World

AT*M2M*NET_DISCONN=0

<- *M2M*NET_DISCONN:OK

<- *M2M*NET_DISCONN:DONE 0

// --- TLS client with a client certificate (mutual TLS) ---

// download the client certificate+key bundle once, then reference it by name

AT*M2M*NET_HTTPDOWNLOAD="https://provision.example.com/certs/client01.pem","client01.pem"

<- *M2M*NET_HTTPDOWNLOAD:OK

<- *M2M*NET_HTTPDOWNLOAD:DONE

AT*M2M*NET_CONN=0 ssl 192.168.0.10 8443 0 0 client01.pem

<- *M2M*NET_CONN:OK

<- *M2M*NET_CONN:DONE 0

// --- TLS server ---

// the server certificate+key bundle must be downloaded first

AT*M2M*NET_HTTPDOWNLOAD="https://provision.example.com/certs/server01.pem","server01.pem"

<- *M2M*NET_HTTPDOWNLOAD:OK

<- *M2M*NET_HTTPDOWNLOAD:DONE

AT*M2M*NET_SERVER=1 ssl 8443 server01.pem

<- *M2M*NET_SERVER:OK

<- *M2M*NET_CLI_ACCEPTED:1 192.168.0.111 51000

<- *M2M*NET_RECV:1 192.168.0.111 51000 5 Hello

AT*M2M*NET_SEND=1 5 World

<- *M2M*NET_SEND:OK

8.5 Entering and Leaving Data Mode

// open a TCP connection first

AT*M2M*NET_CONN=0 tcp 192.168.0.2 60000 0 0

<- *M2M*NET_CONN:OK

<- *M2M*NET_CONN:DONE 0

// switch link 0 into passthrough (transparent) mode

AT*M2M*NET_DTMODE=1 0

<- *M2M*NET_DTMODE:OK 1 0

// ... UART bytes are now relayed directly to link 0 ...

// leave passthrough mode: at least 20 ms of silence, then

+++

// ...and at least 20 ms more silence; the AT parser is active again

AT

<- OK

8.6 MQTT Broker and AWS IoT

The MQTT broker example configures and connects a generic broker; the AWS IoT example shows the certificate-provisioning pipeline used the first time a device connects.

// --- generic MQTT broker ---

AT*M2M*MQTT_CONF=0 0 "iot.example.com" 1883 "" "device01" "" "" ""

<- *M2M*MQTT_CONF:OK

AT*M2M*MQTT_CONN=0

<- *M2M*MQTT_CONN:OK

// then, asynchronously:

<- *M2M*MQTT_CONN:IND 0 0 iot.example.com 1883  1

<- *M2M*MQTT_CONN:DONE

AT*M2M*MQTT_SUB=0 0 "device01/cmd" 1

<- *M2M*MQTT_SUB:OK

AT*M2M*MQTT_PUB=0 "device01/status" "{""power"":""on""}" 0 0

<- *M2M*MQTT_PUB:OK

<- *M2M*MQTT_RECV:IND 0 device01/cmd 16 {"power":"off"}

AT*M2M*MQTT_CLEAN=0

<- *M2M*MQTT_CLEAN:OK

<- *M2M*MQTT_CLEAN:DONE

// --- AWS IoT Core, first connection (no certificate yet) ---

AT*M2M*AWS_PAIR="AB12-CD34-EF56"

<- *M2M*AWS_PAIR:OK

<- *M2M*AWS_PAIR:DONE "https://provision.example.com/v1/device"

AT*M2M*AWS_CONN=1

<- *M2M*AWS_CONN:OK

<- *M2M*AWS_CONN:IND PROVISIONING 1

<- *M2M*AWS_CONN:DONE 1

AT*M2M*AWS_SUB=1 "device-0123/cmd" 1

<- *M2M*AWS_SUB:OK

AT*M2M*AWS_PUB=1 "device-0123/telemetry" 1 0 21

> {"temp":23.5,"hum":41}

<- *M2M*AWS_PUB:OK

<- *M2M*AWS_PUB:DONE 1

AT*M2M*AWS_DISC=1

<- *M2M*AWS_DISC:OK

<- *M2M*AWS_DISC:DONE 1

Appendix A. CONF Reference

AT*M2M*SYS_CONF reads a single attribute by index; AT*M2M*SYS_CONF writes the subset of attributes marked writable. Indexes not listed are reserved.

A.1 Readable Attributes (AT*M2M*SYS_CONF)

| Index | Attribute | Value |
| --- | --- | --- |
| 0 | MAC address | e.g. 00:12:34:56:78:90 |
| 1 | Serial Number | String (32 bytes) |
| 3~32 | Reserved | N/A |
| 33 | Company | Company Code (2 bytes) |
| 34 | Company version | Company Version (2 bytes) |
| 35 | Device code | 0–65535 |
| 36 | Device type | 0–65535 |
| 37 | Device name | string, 16 bytes |
| 38 | Model name | String |
| 39 | Manufacturer | String |

A.2 Writable Attributes (AT*M2M*SYS_CONF)

| Index | Attribute | Parameters |
| --- | --- | --- |
| 35 | Device code | 0–65535 |
| 36 | Device type | 0–65535 |
| 37 | Device name | string, 16 bytes |
| 38 | Model name | String |
| 39 | Manufacturer | String |

Appendix B. Country Codes and Channel Plans

AT*M2M*COUNTRY selects the regulatory domain, which in turn limits the usable 2.4 GHz and 5 GHz channels as follows. 5 GHz channel lists are given as inclusive ranges in steps of 4.

| Code | Country / Region | 2.4 GHz | 5 GHz |
| --- | --- | --- | --- |
| AU | Australia | 1–13 | 36–64, 100–140, 149–165 |
| CA | Canada | 1–11 | 36–64, 100–140, 149–165 |
| CN | China | 1–13 | 36–64, 149–165 |
| EU | EU | 1–13 | 36–64, 100–140 |
| GB | United Kingdom | 1–13 | 36–64, 100–132, 140 |
| IN | India | 1–13 | 36–64, 100–140 |
| JP | Japan | 1–14 | 36–64, 100–140 |
| KR | Republic of Korea | 1–13 | 36–64, 100–128, 149–165 |
| NZ | New Zealand | 1–13 | 36–64, 100–140, 149–165 |
| SG | Singapore | 1–13 | 36–64, 132–140, 149–161 |
| TW | Taiwan | 1–13 | 56–64, 100–140, 149–161 |
| US | United States | 1–11 | 36–64, 100–140, 149–165 |

Appendix C. Error Codes

When a command fails, the module answers *M2M*<NAME>:ERROR followed by one of the codes below..

| Code | Identifier | Meaning |
| --- | --- | --- |
| 1 | ERR_SOCKET_NOT_AVAIL | No free socket of the requested type. |
| 2 | ERR_SOCKET_INVALID | The descriptor is not valid. |
| 3 | ERR_SOCKET_NOT_EXIST | No socket with this descriptor exists. |
| 4 | ERR_CONNECTION_ESTABLISHMENT | The connection could not be established. |
| 5 | ERR_WIFI_CONFIG_PARAM_INVALID | Invalid Wi-Fi configuration parameter. |
| 6 | ERR_TCPIP_PARAM_INVALID | Invalid TCP/IP parameter. |
| 7 | ERR_OUT_OF_MEMORY | Memory allocation failed. |
| 8 | ERR_GENERAL_PARAM_INVALID | Invalid parameter. |
| 9 | ERR_COMMAND_NOT_EXIST | Unknown command. |
| 10 | ERR_ADDRESS_IN_USE | The address or port is already in use. |